American Battery Solutions Inc.

| S32K344-MCAL User Manual |  |
| --- | --- |
| Document ID: N/A | Release Date: 1/14/2025 |
| Revision: 1.5 | Owner: ABS Software Team |

(c) [2019] [American Battery Solutions]; all rights reserved. This document may contain proprietary information and may only be released to third parties with approval of management. Documents are uncontrolled unless otherwise marked; uncontrolled documents are not subject to update notification.

Table of Contents

1	Scope of this Document	5

2	Important Terms	6

3	Functional Overview	7

3.1	Block Diagram	8

3.2	Dependency	9

3.3	Interfaces	10

3.3.1	AUTOSAR Fee	11

3.3.1.1	Fee_Read	12

3.3.1.2	Fee_Write	13

3.3.1.3	Fee_Cancel	14

3.3.1.4	Fee_GetStatus	15

3.3.1.5	Fee_GetJobResult	16

3.3.1.6	Fee_InvalidateBlock	17

3.3.1.7	Fee_GetVersionInfo	18

3.3.1.8	Fee_EraseImmediateBlock	19

3.3.1.9	Fee_GetRunTimeInfo	20

3.3.1.10	Fee_ForceSwapOnNextWrite	21

3.3.2	Cache	22

3.3.2.1	Cache_Ip_Enable	23

3.3.2.2	Cache_Ip_Disable	24

3.3.2.3	Cache_Ip_Invalidate	25

3.3.2.4	Cache_Ip_Clean	26

3.3.2.5	Cache_Ip_InvalidateByAddr	27

3.3.2.6	Cache_Ip_CleanByAddr	28

3.3.3	Enhanced Direct Memory Access (eDMA)	29

3.3.3.1	Dma_Ip_SetLogicInstanceCommand	30

3.3.3.2	Dma_Ip_GetLogicInstanceStatus	31

3.3.3.3	Dma_Ip_LogicChannelInit	32

3.3.3.4	Dma_Ip_LogicChannelDeinit	33

3.3.3.5	Dma_Ip_SetLogicChannelCommand	34

3.3.3.6	Dma_Ip_GetLogicChannelStatus	35

3.3.3.7	Dma_Ip_SetLogicChannelGlobalList	36

3.3.3.8	Dma_Ip_SetLogicChannelTransferList	37

3.3.3.9	Dma_Ip_SetLogicChannelScatterGatherList	39

3.3.3.10	Dma_Ip_GetLogicChannelParam	40

3.3.3.11	Dma_Ip_SetLogicChannelScatterGatherConfig	41

3.3.3.12	Dma_Mux_Ip_Init	42

3.3.4	FlexCAN	43

3.3.4.1	FlexCAN_Ip_SetRxMaskType	44

3.3.4.2	FlexCAN_Ip_SetRxIndividualMask	45

3.3.4.3	FlexCAN_Ip_SetStartMode	46

3.3.4.4	FlexCAN_Ip_SetStopMode	47

3.3.4.5	FlexCAN_Ip_EnterFreezeMode	48

3.3.4.6	FlexCAN_Ip_ExitFreezeMode	49

3.3.4.7	FlexCAN_Ip_ConfigRxMb	50

3.3.4.8	FlexCAN_Ip_Init	51

3.3.4.9	FlexCAN_Ip_Deinit	52

3.3.4.10	FlexCAN_Ip_Receive	53

3.3.4.11	FlexCAN_Ip_Send	54

3.3.4.12	FlexCAN_Ip_MainFunctionWrite	55

3.3.4.13	FlexCAN_Ip_ClearBuffStatusFlag	56

3.3.4.14	FlexCAN_Ip_EnableInterrupts	57

3.3.4.15	FlexCAN_Ip_ConfigRxFifo	58

3.3.4.16	FlexCAN_Ip_RxFifo	58

3.3.5	Interrupt Controller (IntCtrl)	59

3.3.5.1	IntCtrl_Ip_Init	60

3.3.5.2	IntCtrl_Ip_InstallHandler	61

3.3.5.3	IntCtrl_Ip_EnableIrq	62

3.3.5.4	IntCtrl_Ip_DisableIrq	63

3.3.5.5	IntCtrl_Ip_SetPriority	64

3.3.5.6	IntCtrl_Ip_GetPriority	65

3.3.5.7	IntCtrl_Ip_ClearPending	66

3.3.5.8	IntCtrl_Ip_SetPending	67

3.3.5.9	IntCtrl_Ip_GetPending	68

3.3.5.10	IntCtrl_Ip_GetActive	69

3.3.6	Low-Power SPI (LPSPI)	70

3.3.6.1	Lpspi_Ip_SyncTransmit	71

3.3.6.2	Lpspi_Ip_AsyncTransmit	72

3.3.6.3	Lpspi_Ip_GetStatus	73

3.3.6.4	Lpspi_Ip_ManageBuffers	74

3.3.6.5	Lpspi_Ip_UpdateTransferParam	75

3.3.6.6	Lpspi_Ip_UpdateFrameSize	76

3.3.6.7	Lpspi_Ip_UpdateLsb	77

3.3.6.8	Lpspi_Ip_UpdateByteSwap	78

3.3.6.9	Lpspi_Ip_UpdateDefaultTransmitData	79

3.3.6.10	Lpspi_Ip_UpdateTransferMode	80

3.3.6.11	Lpspi_Ip_Cancel	81

3.3.7	System Integration Unit Lite2 (SIUL2)	82

3.3.7.1	Siul2_Port_Ip_SetPullSel	83

3.3.7.2	Siul2_Port_Ip_SetOuptutBuffer	84

3.3.7.3	Siul2_Port_Ip_SetInputBuffer	85

3.3.7.4	Siul2_Port_Ip_Init	86

3.3.7.5	Siul2_Port_Ip_SetPinDirection	87

3.3.7.6	Siul2_Port_Ip_RevertPinConfiguration	88

3.3.7.7	Siul2_Port_Ip_GetPinConfiguration	89

3.3.7.8	Siul2_Dio_Ip_WritePin	90

3.3.7.9	Siul2_Dio_Ip_SetGPDO	91

3.3.7.10	Siul2_Dio_Ip_ClearGPDO	92

3.3.7.11	Siul2_Dio_Ip_WritePins	93

3.3.7.12	Siul2_Dio_Ip_GetPinsOutput	94

3.3.7.13	Siul2_Dio_Ip_SetPins	95

3.3.7.14	Siul2_Dio_Ip_ClearPins	96

3.3.7.15	Siul2_Dio_Ip_TogglePins	97

3.3.7.16	Siul2_Dio_Ip_ReadPins	98

3.3.7.17	Siul2_Dio_Ip_MaskedWritePins	99

3.3.7.18	Siul2_Dio_Ip_ReadPin	100

3.3.7.19	Siul2_Icu_Ip_DeInit	101

3.3.7.20	Siul2_Icu_Ip_Initi	102

3.3.7.21	Siul2_Icu_Ip_SetActivationCondition	103

3.3.7.22	Siul2_Icu_Ip_GetInputState	104

3.3.7.23	Siul2_Icu_Ip_Enable Interrupt	105

3.3.7.24	Siul2_Icu_Ip_DisableInterrupt	106

3.3.7.25	Siul2_Icu_Ip_SetClockMode	107

3.3.7.26	Siul2_Icu_Ip_EnableNotification	108

3.3.7.27	Siul2_Icu_Ip_DisableNotification	109

3.3.8	ADC Successive Approximation Register (SAR)	111

3.3.8.1	Adc_Sar_Ip_GetDataAddress	112

3.3.8.2	Adc_Sar_Ip_ChainConfig	113

3.3.8.3	Adc_Sar_Ip_EnableChannel	114

3.3.8.4	Adc_Sar_Ip_DisableChannel	115

3.3.8.5	Adc_Sar_Ip_SetResolution	116

3.3.8.6	Adc_Sar_Ip_StartConversion	117

3.3.8.7	Adc_Sar_Ip_GetStatusFlags	118

3.3.8.8	Adc_Sar_Ip_ClearStatusFlags	119

3.3.8.9	Adc_Sar_Ip_GetConvDataToArray	120

3.3.8.10	Adc_Sar_Ip_GetConvResultsToArray	121

3.3.8.11	Adc_Sar_Ip_GetConvData	122

3.3.8.12	Adc_Sar_Ip_GetConvResult	123

3.3.8.13	Adc_Sar_Ip_DoCalibration	124

3.3.8.14	Adc_Sar_Ip_Powerup	125

3.3.8.15	Adc_Sar_Ip_Powerdown	126

3.3.8.16	Adc_Sar_Ip_EnableNotifications	127

3.3.8.17	Adc_Sar_Ip_DisableNotifications	128

3.3.8.18	Adc_Sar_Ip_EnableChannelNotifications	129

3.3.8.19	Adc_Sar_Ip_DisableChannelNotifications	130

3.3.8.20	Adc_Sar_Ip_SetClockMode	131

3.3.8.21	Adc_Sar_Ip_SetSampleTimes	132

3.3.8.22	Adc_Sar_Ip_SetAveraging	133

3.3.8.23	Adc_Sar_Ip_AbortConversion	134

3.3.8.24	Adc_Sar_Ip_AbortChain	135

3.3.8.25	Adc_Sar_Ip_SetPresamplingSource	136

3.3.8.26	Adc_Sar_Ip_EnableChannelPresampling	137

3.3.8.27	Adc_Sar_Ip_DisableChannelPresampling	138

3.3.8.28	Adc_Sar_Ip_EnablePresampleConversion	139

3.3.8.29	Adc_Sar_Ip_DisablePresampleConversion	140

3.3.8.30	Adc_Sar_Ip_EnableDma	141

3.3.8.31	Adc_Sar_Ip_DisableDma	142

3.3.8.32	Adc_Sar_Ip_EnableChannelDma	143

3.3.8.33	Adc_Sar_Ip_DisableChannelDma	144

3.3.8.34	Adc_Sar_Ip_DisableChannelDmaAll	145

3.3.8.35	Adc_Sar_Ip_SetDmaClearSource	146

3.3.8.36	Adc_Sar_Ip_SetWdgThreshold	147

3.3.8.37	Adc_Sar_Ip_SetConversionMode	148

3.3.8.38	Adc_Sar_Ip_SetCtuMode	149

3.3.8.39	Adc_Sar_Ip_SetExternalTrigger	150

3.3.8.40	Adc_Sar_Ip_SetUserGainAndOffset	151

3.3.9	Periodic Interrupt Timer	152

3.3.9.1	Pit_Ip_GetInterruptStatusFlag	153

3.3.9.2	Pit_Ip_GetLoadValue	154

3.3.9.3	Pit_Ip_Init	155

3.3.9.4	Pit_Ip_InitChannel	156

3.3.9.5	Pit_Ip_Deinit	157

3.3.9.6	Pit_Ip_StartChannel	158

3.3.9.7	Pit_Ip_StopChannel	159

3.3.9.8	Pit_Ip_GetCurrentTimer	160

3.3.9.9	Pit_Ip_EnableChannelInterrupt	161

3.3.9.10	Pit_Ip_DisableChannelInterrupt	162

3.3.9.11	Pit_Ip_SetLifetimeTimer	163

3.3.9.12	Pit_Ip_GetLifetimeTimer	164

3.4	Configuration	165

3.4.1	AUTOSAR Fee	166

3.4.2	Cache Ip	169

3.4.3	Dma Ip	170

3.4.4	IntCtrl Ip	174

3.4.5	FlexCAN Ip	175

3.4.5.1	FlexCAN Device name	175

3.4.5.2	FlexCAN Device 0 configuration	175

3.4.5.3	FlexCAN Device 1 configuration	177

3.4.5.4	FlexCAN Device 2 configuration	178

3.4.5.5	FlexCAN General	179

3.4.5.6	FlexCAN Timeout Configuration	179

3.4.6	Lpspi Ip	179

3.4.7	Rm	182

3.4.8	Siul2 Dio	185

3.4.9	Siul2 Port	186

3.4.10	Siul2_Icu	197

3.4.11	Pit	200

3.4.12	ADC SAR	202

3.4.13	Emios Mcl Ip	206

3.4.14	Emios PWM	207

3.5	Calibrations	209

3.6	Sequence Diagrams	210

4	Source File List and Description	211

4.1	Source Files	211

4.1.1	Directory: board	211

4.1.2	Directory: generate	212

4.1.3	Directory: mcal-base	212

4.1.4	Directory: Project_Settings	212

4.1.5	Directory: RTD	212

4.2	CMakeLists.txt	212

5	Build and Test	213

6	Integration Procedure	216

7	Resource Consumption	217

8	MISRA + BARR-C	218

9	References	219

Scope of this Document

This document outlines a detailed description of the S32K344-MCAL software component. This outlines design details, key features, functionalities, and how to integrate this into an application.

Important Terms

None

Functional Overview

The S32K344-MCAL component provides application software with access to APIs that drive the MCU peripherals for a given target application.

Block Diagram

Dependency

This component requires the software components to be integrated alongside to be functional.

- absdef (https://bitbucket.org/americanbatterysolutions/absdef)

- S32K344-FreeRTOS (https://bitbucket.org/americanbatterysolutions/s32k344-freertos)

Interfaces

The following section will provide more details for the interfaces of the S32K344-MCAL component.  The details include the function prototype, parameters, and return value description.

AUTOSAR Fee

Below is a component diagram of the AUTOSAR Fee:

Fee_Read

Prototype:

Std_ReturnType Fee_Read(uint16 BlockNumber, uint16 BlockOffset, uint8 *DataBufferPtr, uint16 Length);

Description:

/**

* @brief        Service to initiate a read job.

* @details      The function Fee_Read shall take the block start address and

*               offset and calculate the corresponding memory read address.

*

* @param[in]    BlockNumber     Number of logical block, also denoting

*                                start address of that block in flash memory.

* @param[in]    BlockOffset     Read address offset inside the block.

* @param[out]   DataBufferPtr   Pointer to data buffer.

* @param[in]    Length          Number of bytes to read.

*

* @pre          The module must be initialized, not busy, uBlockNumber must be

*               valid, uLength != 0, pDataBufferPtr != NULL_PTR, uBlockOffset and

*               (uBlockOffset + uLength - 1) must be in range Length must be aligned with FEE_MINIMUM_READ_PAGE_SIZE.

* @post         changes Fee_eModuleStatus module status and Fee_uJobBlockOffset,

*               Fee_uJobBlockLength, Fee_uJobBlockIndex, Fee_pJobReadDataDestPtr,

*               Fee_eJob, Fee_eJobResult job control internal variables.

*

* @return       Std_ReturnType

* @retval       E_OK             The read job was accepted by the

*                                underlying memory driver.

* @retval       E_NOT_OK         The read job has not been accepted

*                                by the underlying memory driver.

* @note         The function Autosar Service ID[hex]: 0x02.

* @note         Asynchronous.

* @note         Non Reentrant.

*

* @api

*

* @implements   Fee_Read_Activity

*/

Fee_Write

Prototype:

Std_ReturnType Fee_Write(uint16 BlockNumber, const uint8 *DataBufferPtr);

Description:

/**

* @brief        Service to initiate a write job.

* @details      The function Fee_Write shall take the block start address and

*               calculate the corresponding memory write address. The block

*               address offset shall be fixed to zero.

*               The function Fee_Write shall copy the given or computed

*               parameters to module internal variables, initiate a write job,

*               set the FEE module status to MEMIF_BUSY, set the job result to

*               MEMIF_JOB_PENDING and return with E_OK.The FEE module shall

*               execute the write job of the function Fee_Write

*               asynchronously within the FEE module' s main function.

*

* @param[in]    BlockNumber     Number of logical block, also denoting

*                                start address of that block in emulated

*                                EEPROM.

* @param[out]   DataBufferPtr   Pointer to data buffer.

*

* @return       Std_ReturnType

* @retval       E_OK             The write job was accepted by the

*                                underlying memory driver.

* @retval       E_NOT_OK         The write job has not been accepted by

*                                the underlying memory driver.

*

* @pre          The module must be initialized, not busy, uBlockNumber must be

*               valid, and pDataBufferPtr != NULL_PTR must be aligned with FEE_BUFFER_ALIGN_VALUE.

*               Before call the function "Fee_Write" for immediate date must be

*               called the function "Fee_EraseImmediateBlock".

* @post         changes Fee_eModuleStatus module status and Fee_uJobBlockIndex,

*               Fee_pJobWriteDataDestPtr, Fee_eJob, Fee_eJobResult job control internal

*               variables.

* @note         The function Autosar Service ID[hex]: 0x03.

* @note         Asynchronous.

* @note         Non Reentrant.

*

* @api

*

* @implements   Fee_Write_Activity

*/

Fee_Cancel

Prototype:

void Fee_Cancel(void);

Description:

/**

* @brief        Service to call the cancel function of the underlying flash

*               driver.

* @details      The function Fee_Cancel and the cancel function of the

*               underlying flash driver are asynchronous w.r.t. an ongoing

*               read, erase or write job in the flash memory.

*

* @pre          The module must be initialized.

* @post         Changes Fee_eModuleStatus module status and job result

*               Fee_eJobResult internal variables.

* @note         The function Autosar Service ID[hex]: 0x04.

* @note         Synchronous.

* @note         Non Reentrant.

*

* @api

*

* @implements   Fee_Cancel_Activity

*/

Fee_GetStatus

Prototype:

MemIf_StatusType Fee_GetStatus(void);

Description:

/**

* @brief        Return the Fee module state.

* @details      Return the Fee module state synchronously.

*

* @note         The function Autosar Service ID[hex]: 0x05.

* @note         Synchronous

* @note         Non Reentrant

*

* @return       Fee_eModuleStatus

* @retval       MEMIF_UNINIT             Module has not been initialized (yet).

* @retval       MEMIF_IDLE               Module is currently idle.

* @retval       MEMIF_BUSY               Module is currently busy.

* @retval       MEMIF_BUSY_INTERNAL      Module is busy with internal management

*                                        operations.

*

* @api

*

* @implements   Fee_GetStatus_Activity

*/

Fee_GetJobResult

Prototype:

MemIf_JobResultType Fee_GetJobResult(void);

Description:

/**

* @brief        Return the result of the last job.

* @details      Return the result of the last job synchronously.

*

* @return       MemIf_JobResultType

* @retval       MEMIF_JOB_OK              The job has been finished successfully.

* @retval       MEMIF_JOB_FAILED          The job has not been finished

*                                         successfully.

* @retval       MEMIF_JOB_PENDING         The job has not yet been finished.

* @retval       MEMIF_JOB_CANCELED        The job has been canceled.

* @retval       MEMIF_BLOCK_INCONSISTENT  The requested block is inconsistent,

*                                         it may contain corrupted data.

* @retval       MEMIF_BLOCK_INVALID       The requested block has been invalidated,

*                                         the requested read operation can not be performed.

* @note         The function Autosar Service ID[hex]: 0x06.

* @note         Synchronous.

* @note         Non Reentrant.

*

* @api

*

* @implements   Fee_GetJobResult_Activity

*/

Fee_InvalidateBlock

Prototype:

Std_ReturnType Fee_InvalidateBlock(uint16 BlockNumber);

Description:

/**

* @brief        Service to invalidate a logical block.

*

* @param[in]    BlockNumber     Number of logical block, also denoting

*                                start address of that block in flash memory

* @return       Std_ReturnType

* @retval       E_OK             The job was accepted by the underlying

*                                memory driver.

* @retval       E_NOT_OK         The job has not been accepted by the

*                                underlying memory driver.

*

* @pre          The module must be initialized, not busy, and uBlockNumber must be

*               valid

*

* @post         changes Fee_eModuleStatus module status and Fee_uJobBlockIndex,

*               Fee_eJob, and Fee_eJobResult job control internal variables.

*               EEPROM.

* @note         The function Autosar Service ID[hex]: 0x07.

* @note         Asynchronous.

* @note         Non Reentrant.

*

* @api

*

* @implements   Fee_InvalidateBlock_Activity

*/

Fee_GetVersionInfo

Prototype:

void Fee_GetVersionInfo(Std_VersionInfoType *VersionInfoPtr);

Description:

/**

* @brief        Return the version information of the Fee module.

* @details      The version information includes: Module Id, Vendor Id,

*               Vendor specific version numbers.

*

* @param[out]   VersionInfoPtr  Pointer to where to store the version

*                                information of this module .

*

* @pre          pVersionInfoPtr must not be NULL_PTR.

* @note         The function Autosar Service ID[hex]: 0x08.

* @note         Synchronous.

* @note         Non Reentrant.

*

* @api

*

* @implements   Fee_GetVersionInfo_Activity

*/

Fee_EraseImmediateBlock

Prototype:

Std_ReturnType Fee_EraseImmediateBlock(uint16 BlockNumber);

Description:

/**

* @brief        Service to erase a logical block.

* @details      The function Fee_EraseImmediateBlock shall take the block number

*               and calculate the corresponding memory block address.

*               The function Fee_EraseImmediateBlock shall ensure that the FEE

*               module can write immediate data. Whether this involves

*               physically erasing a memory area and therefore calling the erase

*               function of the underlying driver depends on the implementation.

*               If development error detection for the FEE module is enabled,

*               the function Fee_EraseImmediateBlock shall check whether the

*               addressed logical block is configured as containing immediate

*               data (configuration parameter FeeImmediateData == TRUE).

*               If not, the function Fee_EraseImmediateBlock

*               shall report the error code FEE_E_INVALID_BLOCK_NO.

*

* @param[in]    BlockNumber     Number of logical block, also denoting.

*

* @return       Std_ReturnType

* @retval       E_OK             The job was accepted by the underlying

*                                memory driver.

* @retval       E_NOT_OK         The job has not been accepted by the underlying

*                                memory driver.

*                                start address of that block in emulated EEPROM.

*

* @pre          The module must be initialized, not busy, uBlockNumber must be

*               valid, and type of Fee block must be immediate.

* @post         changes Fee_eModuleStatus module status and Fee_uJobBlockIndex,

*               Fee_eJob, and Fee_eJobResult job control internal variables.

*

* @note         The function Autosar Service ID[hex]: 0x09.

* @note         Asynchronous.

* @note         Non Reentrant.

*

* @api

*

* @implements   Fee_EraseImmediateBlock_Activity

*/

Fee_GetRunTimeInfo

Prototype:

void Fee_GetRunTimeInfo(uint8 ClrGrpIndex, Fee_ClusterGroupRuntimeInfoType *ClrGrpRTInfo);

Description:

/**

* @brief        Service to read runtime information in the selected cluster.

*

* @param[in]    ClrGrpIndex    Index of the selected cluster group

* @param[in]    ClrGrpRTInfo   Pointer to point Fee cluster group run-time Information

*

* @return       Fee_ClusterGroupRuntimeInfoType

* @retval       ClusterTotalSpace current cluster total size

* @retval       ClusterFreeSpace current cluster available space.

* @retval       BlockHeaderOverhead the block header overhead (blk header, valid flag and inval flag),

* @retval       VirtualPageSize the virtual page size (the size of an allocation unit)

* @retval       NumberOfSwap number of cluster swap already performed

* @retval       SectorInfo sector information from all clusters in the selected cluster group (in sector retirement mode only)

*

* @pre          The module must be initialized, not busy and uClrGrpIndex must be valid

*

* @api

*

* @implements   Fee_GetRunTimeInfo_Activity

*/

Fee_ForceSwapOnNextWrite

Prototype:

Std_ReturnType Fee_ForceSwapOnNextWrite(uint8 ClrGrpIndex);

Description:

/**

* @brief        Service to prepare the driver for a cluster swap in the selected cluster group.

*

* @detailes     While the computed amount of memory is allocated as a result of Fee_Write

*               call for plain data blocks, for immediate data blocks memory gets completely

*               pre-allocated through Fee_EraseImmediateBlock function (i.e. Fee_Write does

*               not change the remaining space).

*               As a result, swaps triggered by the planned Fee_ForceSwapOnNextWrite function

*               behave the same way, or in other words, an operation that really activates the

*               pysical swap must be either Fee_Write on plain FEE block or

*               Fee_EraseImmediateBlock on immediate data block.

*

* @param[in]    ClrGrpIndex    Index of the selected cluster group

* @return       Std_ReturnType

* @retval       E_NOT_OK  module is not initialized, busy or uClrGrpIndex is not in the

*                         valid range.

* @retval       E_OK      No more space available in the selected cluster.

*

* @pre          The module must be initialized, not busy and uClrGrpIndex must be valid.

*

* @note         As this API manipulates the internal driver state, it has to be claimed

*               non-reentrant and colliding with other FEE ASR APIs

* @api

*

* @implements   Fee_ForceSwapOnNextWrite_Activity

*/

Cache

Below is an illustration of the Cache (sub) component of the MCAL component:

Cache_Ip_Enable

Prototype:

Std_ReturnType Cache_Ip_Enable(const Cache_Ip_Type CacheType,

const Cache_Ip_BusType BusType);

Description:

/**

* @brief This function enables the Cache Ip Driver.

* @details This service is a reentrant function that shall enable the Cache Ip driver.

*

* @param[in]  CacheType         Specifies the cache type.

* @param[in]  BusType           Specifies the bus type.

*

* @return   E_OK is returned if the enable action finished correctly.

*           E_NOT_OK is returned if the enable action finished incorrectly.

*

* @implements Cache_Ip_Enable_Activity

* */

Cache_Ip_Disable

Prototype:

Std_ReturnType Cache_Ip_Disable(const Cache_Ip_Type CacheType,

const Cache_Ip_BusType BusType);

Description:

/**

* @brief This function disables the Cache Ip Driver.

* @details This service is a reentrant function that shall disables the Cache Ip driver.

*

* @param[in]  CacheType         Specifies the cache type.

* @param[in]  BusType           Specifies the bus type.

*

* @return   E_OK is returned if the disable action finished correctly.

*           E_NOT_OK is returned if the disable action finished incorrectly.

*

* @implements Cache_Ip_Disable_Activity

* */

Cache_Ip_Invalidate

Prototype:

Std_ReturnType Cache_Ip_Invalidate(const Cache_Ip_Type CacheType,

const Cache_Ip_BusType BusType);

Description:

/**

* @brief This function Invalidates the Cache Ip Driver.

* @details This service is a reentrant function that shall Invalidates the Cache Ip driver.

*          The Invalidate operation applies for the entire selected Cache Type.

*

*          A cache invalidate operation ensures that updates made visible by observers that access memory at

*          the point to which the invalidate is defined are made visible to an observer that controls the cache.

*          This might result in the loss of updates to the locations affected by the invalidate operation that have

*          been written by observers that access the cache. If the address of an entry on which the invalidate

*          operates does not have a Normal Cacheable attribute, or if the cache is disabled, then an invalidate

*          operation also ensures that this address is not present in the cache.

*

* @param[in]  CacheType         Specifies the cache type.

* @param[in]  BusType           Specifies the bus type.

*

* @return   E_OK is returned if the invalidation finished correctly.

*           E_NOT_OK is returned if the invalidation finished incorrectly.

*

* @implements Cache_Ip_Invalidate_Activity

* */

Cache_Ip_Clean

Prototype:

Std_ReturnType Cache_Ip_Clean(const Cache_Ip_Type CacheType,

const Cache_Ip_BusType BusType,

const boolean EnInvalidate);

Description:

/**

* @brief This function Clean the Cache Ip Driver.

* @details This service is a reentrant function that shall Clean the Cache Ip driver.

*          The Clean operation applies for the entire selected Cache Type.

*

*          A cache clean operation ensures that updates made by an observer that controls the cache are made

*          visible to other observers that can access memory at the point to which the operation is performed.

*          Once the Clean has completed, the new memory values are guaranteed to be visible to the point to

*          which the operation is performed, for example to the point of unification. The cleaning of a cache

*          entry from a cache can overwrite memory that has been written by another observer only if the entry

*          contains a location that has been written to by an observer in the shareability domain of that memory

*          location.

*

*          A cache clean and invalidate operation behaves as the execution of a clean operation followed

*          immediately by an invalidate operation. Both operations are performed to the same location.

*

* @param[in]  CacheType         Specifies the cache type.

* @param[in]  BusType           Specifies the bus type.

* @param[in]  enInvalidate      Specifies to execute operation Clean&Invalidate.

*

* @return   E_OK is returned if the cleaning finished correctly.

*           E_NOT_OK is returned if the cleaning finished incorrectly.

*

* @implements Cache_Ip_Clean_Activity

* */

Cache_Ip_InvalidateByAddr

Prototype:

Std_ReturnType Cache_Ip_InvalidateByAddr(const Cache_Ip_Type CacheType,

const Cache_Ip_BusType BusType,

const uint32 Addr,

const uint32 Length);

Description:

/**

* @brief This function Invalidates By Address the Cache Ip Driver.

* @details This service is a reentrant function that shall Invalidates By Address the Cache Ip driver.

*          The Invalidate By Address operation applies for the memory segment specified by the

*          start Address and Length. The operation Invalidates only multiple of Cache Line Size, thus the

*          specified memory segment shall be aligned and multiple of the Cache Line Size.

*

*          A cache invalidate operation ensures that updates made visible by observers that access memory at

*          the point to which the invalidate is defined are made visible to an observer that controls the cache.

*          This might result in the loss of updates to the locations affected by the invalidate operation that have

*          been written by observers that access the cache. If the address of an entry on which the invalidate

*          operates does not have a Normal Cacheable attribute, or if the cache is disabled, then an invalidate

*          operation also ensures that this address is not present in the cache.

*

* @param[in]  CacheType         Specifies the cache type.

* @param[in]  BusType           Specifies the bus type.

* @param[in]  Addr              Specifies the memory segment start address.

* @param[in]  Length            Specifies the memory segment length.

*

* @return   E_OK is returned if the invalidation finished correctly.

*           E_NOT_OK is returned if the invalidation finished incorrectly.

*

* @implements Cache_Ip_InvalidateByAddr_Activity

* */

Cache_Ip_CleanByAddr

Prototype:

Std_ReturnType Cache_Ip_CleanByAddr(const Cache_Ip_Type CacheType,

const Cache_Ip_BusType BusType,

const boolean EnInvalidate,

const uint32 Addr,

const uint32 Length);

Description:

/**

* @brief This function Clean By Address the Cache Ip Driver.

* @details This service is a reentrant function that shall Clean By Address the Cache Ip driver.

*          The Clean By Address operation applies for the memory segment specified by the

*          start Address and Length. The operation Cleans only multiple of Cache Line Size, thus the

*          specified memory segment shall be aligned and multiple of the Cache Line Size.

*

*          A cache clean operation ensures that updates made by an observer that controls the cache are made

*          visible to other observers that can access memory at the point to which the operation is performed.

*          Once the Clean has completed, the new memory values are guaranteed to be visible to the point to

*          which the operation is performed, for example to the point of unification. The cleaning of a cache

*          entry from a cache can overwrite memory that has been written by another observer only if the entry

*          contains a location that has been written to by an observer in the shareability domain of that memory

*          location.

*

*          A cache clean and invalidate operation behaves as the execution of a clean operation followed

*          immediately by an invalidate operation. Both operations are performed to the same location.

*

* @param[in]  CacheType         Specifies the cache type.

* @param[in]  BusType           Specifies the bus type.

* @param[in]  enInvalidate      Specifies to execute operation Clean&Invalidate.

* @param[in]  Addr              Specifies the memory segment start address.

* @param[in]  Length            Specifies the memory segment length.

*

* @return   E_OK is returned if the cleaning finished correctly.

*           E_NOT_OK is returned if the cleaning finished incorrectly.

*

* @implements Cache_Ip_CleanByAddr_Activity

* */

Enhanced Direct Memory Access (eDMA)

Below is an illustration of the eDMA  and DMA MUX (sub) components of the MCAL component:

Dma_Ip_SetLogicInstanceCommand

Prototype:

Dma_Ip_ReturnType Dma_Ip_SetLogicInstanceCommand(const uint32 LogicInst,

const Dma_Ip_LogicInstanceCmdType Command);

Description:

/**

* @brief This function sets Dma Ip Instance Command.

* @details This service is a reentrant function that shall command the Dma Instance.

*          The command shall trigger specific functionalities of the Dma Instance.

*

* @param[in]  LogicInst         Selection value of the Logic Instance.

* @param[in]  Command           The command for the Logic Instance.

*

* @return     Dma_Ip_ReturnType DMA_IP_STATUS_SUCCESS is returned if the command finished ok.

*

* @implements Dma_Ip_SetLogicInstanceCommand_Activity

* */

Dma_Ip_GetLogicInstanceStatus

Prototype:

Dma_Ip_ReturnType Dma_Ip_GetLogicInstanceStatus(const uint32 LogicInst,

Dma_Ip_LogicInstanceStatusType * const Status);

Description:

/**

* @brief This function gets Dma Ip Instance Status.

* @details This service is a reentrant function that shall get the Dma Instance status.

*          The command shall read specific functionalities of the Dma Instance.

*

* @param[in]  LogicInst         Selection value of the Logic Instance.

* @param[out] Status            Pointer to the Dma Instance status.

*

* @return Dma_Ip_ReturnType DMA_IP_STATUS_SUCCESS is returned if the get status finished ok.

*

* @implements Dma_Ip_GetLogicInstanceStatus_Activity

* */

Dma_Ip_LogicChannelInit

Prototype:

Dma_Ip_ReturnType Dma_Ip_LogicChannelInit(const uint32 LogicCh);

Description:

/**

* @brief This function initializes the Dma Ip Logic Channel.

* @details This service is a non reentrant function that shall initialize the Dma Ip Logic Channel.

*

* @param[in]  LogicCh           Selection value of the Logic Channel.

*

* @return     Dma_Ip_ReturnType DMA_IP_STATUS_SUCCESS is returned if the initialization finished ok.

*                               DMA_IP_STATUS_WRONG_STATE is returned if the Dma Ip Channel state is

*                               not Dma_Ip_Ch_ResetState.

*

* @implements Dma_Ip_LogicChannelInit_Activity

* */

Dma_Ip_LogicChannelDeinit

Prototype:

Dma_Ip_ReturnType Dma_Ip_LogicChannelDeinit(const uint32 LogicCh);

Description:

/**

* @brief This function deinitializes the Dma Ip Logic Channel.

* @details This service is a non reentrant function that shall deinitialize the Dma Ip Logic Channel.

*

* @param[in]  LogicCh           Selection value of the Logic Channel.

*

* @return     Dma_Ip_ReturnType DMA_IP_STATUS_SUCCESS is returned if the deinitialization finished ok.

*

* @implements Dma_Ip_LogicChannelDeinit_Activity

* */

Dma_Ip_SetLogicChannelCommand

Prototype:

Dma_Ip_ReturnType Dma_Ip_SetLogicChannelCommand(const uint32 LogicCh,

const Dma_Ip_LogicChannelCmdType Command);

Description:

/**

* @brief This function sets Dma Ip Logic Channel Command.

* @details This service is a reentrant function that shall command the Dma Channel.

*          The command shall trigger specific functionalities of the Dma Channel.

*

* @param[in]  LogicCh           Selection value of the Logic Channel.

* @param[in]  Command           The command for the Logic Channel.

*

* @return     Dma_Ip_ReturnType DMA_IP_STATUS_SUCCESS is returned if the initialization finished ok.

*                               DMA_IP_STATUS_WRONG_STATE is returned if the Dma Ip Channel state is

*                               not Dma_Ip_Ch_ReadyState.

*

* @implements Dma_Ip_SetLogicChannelCommand_Activity

* */

Dma_Ip_GetLogicChannelStatus

Prototype:

Dma_Ip_ReturnType Dma_Ip_GetLogicChannelStatus(const uint32 LogicCh,

Dma_Ip_LogicChannelStatusType * const ChStatus);

Description:

/**

* @brief This function gets Dma Ip Logic Channel Status.

* @details This service is a reentrant function that shall get the Dma Channel status.

*          The command shall read specific functionalities of the Dma Channel.

*

* @param[in]  LogicCh           Selection value of the Logic Channel.

* @param[out] Status            Pointer to the Dma Channel status.

*

* @return Dma_Ip_ReturnType DMA_IP_STATUS_SUCCESS is returned if the get status finished ok.

*

* @implements Dma_Ip_GetLogicChannelStatus_Activity

* */

Dma_Ip_SetLogicChannelGlobalList

Prototype:

Dma_Ip_ReturnType Dma_Ip_SetLogicChannelGlobalList(const uint32 LogicCh,

const Dma_Ip_LogicChannelGlobalListType List[],

const uint32 ListDimension);

Description:

/**

* @brief This function sets Dma Ip Logic Channel Global List settings.

* @details This service is a reentrant function that shall set the Dma Ip Logic Channel

*          global parameters list.

*          The list is composed of an array of Dma Ip Logic Channel global parameters settings.

*          The settings list(array) is defined by the user needs: it contains the

*          desired parameters to be configured, in any desired order.

*

*          How to use this interface:

*          1. Use the "Dma_Ip_LogicChannelGlobalListType" to create a list(array) with the desired

*          paramaters to configure (see parameters: "Dma_Ip_LogicChannelGlobalParamType")

*          The list can declared globally or locally:

*          A. Global example:

*              Dma_Ip_LogicChannelGlobalListType global_Dma_Ip_ChannelGlobalList0[NUMBER_OF_PARAMETERS] = {...};

*          B. Local example:

*              Dma_Ip_LogicChannelGlobalListType Dma_Ip_ChannelGlobalList[NUMBER_OF_PARAMETERS];

*              Dma_Ip_ChannelGlobalList[PARAMETER0].Param = DMA_IP_CH_SET_EN_PREEMPTION_PRIORITY;

*              Dma_Ip_ChannelGlobalList[PARAMETER0].Value = TRUE;

*              Dma_Ip_ChannelGlobalList[PARAMETER1].Param = ...;

*              Dma_Ip_ChannelGlobalList[PARAMETER1].Value = ...;

*          2. Call the "Dma_Ip_SetLogicChannelGlobalList()" interface:

*              Dma_Ip_SetLogicChannelGlobalList(LOGIC_CHANNELx, Dma_Ip_ChannelGlobalList, NUMBER_OF_PARAMETERS);

*

* @param[in]  Channel           Specifies the Logic Channel Id.

* @param[in]  List              Pointer to the Global List Array.

* @param[in]  ListDimension     Number of entries in the List.

*

* @return     Dma_Ip_ReturnType DMA_IP_STATUS_SUCCESS is returned if the initialization finished ok.

*                               DMA_IP_STATUS_WRONG_STATE is returned if the Dma Ip Channel state is

*                               not Dma_Ip_Ch_ReadyState.

*

* @implements Dma_Ip_SetLogicChannelGlobalList_Activity

* */

Dma_Ip_SetLogicChannelTransferList

Prototype:

Dma_Ip_ReturnType Dma_Ip_SetLogicChannelTransferList(const uint32 LogicCh,

const Dma_Ip_LogicChannelTransferListType List[],

const uint32 ListDimension);

Description:

/**

* @brief This function sets Dma Ip Logic Channel Transfer List settings.

* @details -> This service is a reentrant function that shall set the Dma Ip Logic Channel

*          transfer parameters list.

*          -> The "Transfer List" loads the configuration directly into the Hardware TCD and disables

*          the ScatterGather for the Hardware TCD.

*          -> The list is composed of an array of Dma Ip Logic Channel transfer parameters settings.

*          -> The settings array is defined by the user needs: it contains entries for each desired

*          parameter to be configured, in any suitable order.

*

*          -> How to use this interface: <-

*          1. Use the "Dma_Ip_LogicChannelTransferListType" to create a list(array) with the desired

*          paramaters to configure (see parameters: "Dma_Ip_LogicChannelTransferParamType")

*          The list can declared globally or locally:

*          Global example:

*              #define DMA_IP_TRANSFER_LIST0_DIMENSION ((uint32)2U)

*              Dma_Ip_LogicChannelTransferListType global_Dma_Ip_ChannelTransferList0[DMA_IP_TRANSFER_LIST0_DIMENSION] = {...};

*          Local example:

*              #define DMA_IP_TRANSFER_LIST0_DIMENSION ((uint32)2U)

*              Dma_Ip_LogicChannelTransferListType Dma_Ip_ChannelTransferList0[DMA_IP_TRANSFER_LIST0_DIMENSION];

*              Dma_Ip_ChannelTransferList0[PARAMETER0].Param = DMA_IP_CH_SET_VAL_SOURCE_ADDRESS;

*              Dma_Ip_ChannelTransferList0[PARAMETER0].Value = &SourceBuffer;

*              Dma_Ip_ChannelTransferList0[PARAMETER1].Param = DMA_IP_CH_SET_DESTINATION_ADDRESS;

*              Dma_Ip_ChannelTransferList0[PARAMETER1].Value = &DestinationBuffer;

*          2. Call the "Dma_Ip_SetLogicChannelTransferList()" interface:

*              Dma_Ip_SetLogicChannelTransferList(LOGIC_CHANNELx, Dma_Ip_ChannelTransferList0, DMA_IP_TRANSFER_LIST0_DIMENSION);

*

*          -> Coding Example: <-

*              -> The user shall create the desired configuration list for his specific application.

*              "UserDefinedFileName.h"

*              #define DMA_IP_TRANSFER_LIST0_DIMENSION ((uint32)8U)

*              #define DMA_IP_SET_TRANSFER_TYPE0(CHANNEL, DIMENSION, SADDR, SOFF, SSIZE, DADDR, DOFF, DSIZE, MINOR_SIZE, MAJOR_COUNT) \

*                          Dma_Ip_LogicChannelTransferListType Dma_Ip_ChannelTransferList0[DIMENSION];                \

*                          Dma_Ip_ChannelTransferList0[0U].Param = DMA_IP_CH_SET_VAL_SOURCE_ADDRESS;                         \

*                          Dma_Ip_ChannelTransferList0[0U].Value = SADDR;                                             \

*                          Dma_Ip_ChannelTransferList0[1U].Param = DMA_IP_CH_SET_SOURCE_SIGNED_OFFSET;                    \

*                          Dma_Ip_ChannelTransferList0[1U].Value = SOFF;                                              \

*                          Dma_Ip_ChannelTransferList0[2U].Param = DMA_IP_CH_SET_SOURCE_TRANSFER_SIZE;                    \

*                          Dma_Ip_ChannelTransferList0[2U].Value = SSIZE;                                             \

*                          Dma_Ip_ChannelTransferList0[3U].Param = DMA_IP_CH_SET_DESTINATION_ADDRESS;                    \

*                          Dma_Ip_ChannelTransferList0[3U].Value = DADDR;                                             \

*                          Dma_Ip_ChannelTransferList0[4U].Param = DMA_IP_CH_SET_DESTINATION_SIGNED_OFFSET;               \

*                          Dma_Ip_ChannelTransferList0[4U].Value = DOFF;                                              \

*                          Dma_Ip_ChannelTransferList0[5U].Param = DMA_IP_CH_SET_DESTINATION_TRANSFER_SIZE;               \

*                          Dma_Ip_ChannelTransferList0[5U].Value = DSIZE;                                             \

*                          Dma_Ip_ChannelTransferList0[6U].Param = DMA_IP_CH_SET_MINORLOOP_SIZE;                         \

*                          Dma_Ip_ChannelTransferList0[6U].Value = MINOR_SIZE;                                        \

*                          Dma_Ip_ChannelTransferList0[7U].Param = DMA_IP_CH_SET_MAJORLOOP_COUNT;                        \

*                          Dma_Ip_ChannelTransferList0[7U].Value = MAJOR_COUNT;                                       \

*                          Dma_Ip_SetLogicChannelTransferList(CHANNEL, Dma_Ip_ChannelTransferList0, DIMENSION);

*

*              "ApplicationFileName.c"

*              void ConfigureDmaChannel(ConfigType * pxConfig)

*              {

*                  uint32 MinorLoopSize = 2U;

*                  uint32 MajorLoopCount;

*                  if(pxConfig->MajorLoopCondition == TRUE)

*                  {

*                      MajorLoopCount = 8U;

*                  }

*                  else

*                  {

*                      MajorLoopCount = 24U;

*                  }

*                  DMA_IP_SET_TRANSFER_TYPE0(pxConfig->LogicChannel,

*                                            DMA_IP_TRANSFER_LIST0_DIMENSION,

*                                            pxConfig->SourceBuffer,

*                                            2U,

*                                            DMA_IP_TRANSFER_SIZE_2_BYTE,

*                                            &RegisterAddress,

*                                            0U,

*                                            DMA_IP_TRANSFER_SIZE_2_BYTE,

*                                            MinorLoopSize,

*                                            MajorLoopCount);

*              }

*

* @param[in]  Channel           Specifies the Logic Channel Id.

* @param[in]  List              Pointer to the Transfer List Array.

* @param[in]  ListDimension     Number of entries in the List.

*

* @return     Dma_Ip_ReturnType DMA_IP_STATUS_SUCCESS is returned if the initialization finished ok.

*                               DMA_IP_STATUS_WRONG_STATE is returned if the Dma Ip Channel state is

*                               not Dma_Ip_Ch_ReadyState.

*

* @implements Dma_Ip_SetLogicChannelTransferList_Activity

* */

Dma_Ip_SetLogicChannelScatterGatherList

Prototype:

Dma_Ip_ReturnType Dma_Ip_SetLogicChannelScatterGatherList(const uint32 LogicCh,

const uint32 Element,

const Dma_Ip_LogicChannelScatterGatherListType List[],

const uint32 ListDimension);

Description:

/**

* @brief This function sets Dma Ip Logic Channel Scatter/Gather List settings.

* @details -> This service is a reentrant function that shall set the Dma Ip Logic Channel

*          scatter/gather parameters list.

*          -> The "Scatter/Gather List" configures Logic Elements belonging to the same

*          Dma Logic Channel.

*          -> The "Scatter/Gather List" loads the configuration into the Software TCD.

*          The Software TCD has the Scatter/Gather Enable set (ESG bit) and the Next Software

*          TCD Address already loaded during the configuration generation process.

*          -> The "Scatter/Gather List" shall not be able to modify the Scatter/Gather Element

*          linkage (reorder of elements in the chain). The linkage of the elements is set only

*          during the configuration process.

*          -> The settings array is defined by the user needs: it contains entries for

*          each desired parameter to be configured, in any suitable order.

*          -> This service does not load the Logic Element into the Hardware TCD. This functionality

*          is covered by Dma_Ip_SetLogicChannelScatterGatherConfig.

*

*          How to use this interface:

*          1. Use the "Dma_Ip_LogicChannelScatterGatherListType" to create a list(array) with the desired

*          paramaters to configure (see parameters: "Dma_Ip_LogicChannelTransferParamType")

*          The list can declared globally or locally:

*          Global example:

*              Dma_Ip_LogicChannelScatterGatherListType global_Dma_Ip_ChannelScatterGatherList0[NUMBER_OF_PARAMETERS] = {...};

*          Local example:

*              Dma_Ip_LogicChannelScatterGatherListType Dma_Ip_ChannelScatterGatherList[NUMBER_OF_PARAMETERS];

*              Dma_Ip_ChannelScatterGatherList[PARAMETER0].Param = DMA_IP_CH_SET_VAL_SOURCE_ADDRESS;

*              Dma_Ip_ChannelScatterGatherList[PARAMETER0].Value = &SourceBuffer;

*              Dma_Ip_ChannelScatterGatherList[PARAMETER1].Param = DMA_IP_CH_SET_DESTINATION_ADDRESS;

*              Dma_Ip_ChannelScatterGatherList[PARAMETER1].Value = &DestinationBuffer;

*          2. Call the "Dma_Ip_SetLogicChannelScatterGatherList()" interface:

*              Dma_Ip_SetLogicChannelScatterGatherList(LOGIC_CHANNELx, LOGIC_ELEMENTy, Dma_Ip_ChannelScatterGatherList, NUMBER_OF_PARAMETERS);

*

* @return     Dma_Ip_ReturnType DMA_IP_STATUS_SUCCESS is returned if the initialization finished ok.

*                               DMA_IP_STATUS_WRONG_STATE is returned if the Dma Ip Channel state is

*                               not Dma_Ip_Ch_ReadyState.

*

* @implements Dma_Ip_SetLogicChannelScatterGatherList_Activity

* */

Dma_Ip_GetLogicChannelParam

Prototype:

Dma_Ip_ReturnType Dma_Ip_GetLogicChannelParam(const uint32 LogicCh,

const Dma_Ip_LogicChannelInfoParamType Param,

uint32 * const Value);

Description:

/**

* @brief This function gets the Dma Ip Logic Channel Parameter value.

* @details This service is a reentrant function that shall get the Dma Channel

*          parameters value.

*

* @param[in]  LogicCh           Selection value of the Logic Channel.

* @param[in]  Param             Selection parameter.

* @param[out] Value             Pointer to the parameter value.

*

* @return Dma_Ip_ReturnType DMA_IP_STATUS_SUCCESS is returned if the get information finished ok.

*

* @implements Dma_Ip_GetLogicChannelParam_Activity

* */

Dma_Ip_SetLogicChannelScatterGatherConfig

Prototype:

Dma_Ip_ReturnType Dma_Ip_SetLogicChannelScatterGatherConfig(const uint32 LogicCh,

const uint32 Element);

Description:

/**

* @brief This function configures the Dma Ip Logic Channel Scatter/Gather.

* @details This service is a reentrant function that shall configure the Dma Channel

*          scatter/gather functionality.

*          The specified Logic Element (corresponding to a Software TCD) shall be loaded

*          into the Dma Logic Channel's Hardware TCD.

*          The Logic Elements (describing the Software TCDs) form a simple chained list

*          and the "Element" parameter represents the lists's head.

*

* @param[in]  LogicCh           Selection value of the Logic Channel.

* @param[in]  Element           Selection value of the Logic Element representing the

*                               list's head.

*

* @return     Dma_Ip_ReturnType DMA_IP_STATUS_SUCCESS is returned if the initialization finished ok.

*                               DMA_IP_STATUS_WRONG_STATE is returned if the Dma Ip Channel state is

*                               not Dma_Ip_Ch_ReadyState.

*                               DMA_IP_STATUS_ERROR is returned if the Dma Ip Channel contains an error.

*

* @implements Dma_Ip_SetLogicChannelScatterGatherConfig_Activity

* */

Dma_Mux_Ip_Init

Prototype:

void Dma_Mux_Ip_Init(const Dma_Mux_Ip_ConfigType * pConfig);

Description:

/**

* @brief         Initializes the Memory Protection Unit general parameters and

*                region configurations

*

* @details       This function is non-reentrant

*

* @param[in]     pConfig: pointer to configuration structure for DMA_MUX.

* @return        void

*

* @pre           None

*

**/

FlexCAN

Below is an illustration of the FlexCAN Controller component provided interfaces:

FlexCAN_Ip_SetRxMaskType

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_SetRxMaskType(instance, type);

Description:

/**

*  @brief      Set RX masking type

*  @details    This function will set RX masking type as RX global mask or RX individual mask

*  @param[in]  instance: A FlexCAN instance number

*  @param[in]  type: FlexCAN Rx mask type

*  @note       This function should be called from StopMode or FreezeMode.

*  @return     FLEXCAN_STATUS_SUCCESS if successful<br>

*              FLEXCAN_STATUS_ERROR if controller is not in freeze mode<br>

*

*/

FlexCAN_Ip_SetRxIndividualMask

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_SetRxIndividualMask(instance, mb_idx, mask);

Description:

/**

*  @brief     Sets the FlexCAN Rx individual mask

*  @details   This function will set directly the mask value as is provided.

*  @param[in] instance A FlexCAN instance number

*  @param[in] mb_idx   Index of the message buffer

*  @param[in] mask     Mask value

*  @note      This function should be called from StopMode or FreezeMode.

*  @return    FLEXCAN_STATUS_SUCCESS if successful;<br>

*             FLEXCAN_STATUS_BUFF_OUT_OF_RANGE if the index of a message buffer is invalid.

*

*/

FlexCAN_Ip_SetStartMode

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_SetStartMode(instance);

Description:

/**

*  @brief     Set the FlexCAN instance in START mode

*  @details   Set the FlexCAN instance in START mode, allowing to participate to bus transfers.

*  @param[in] instance A FlexCAN instance number

*  @return    FLEXCAN_STATUS_SUCCESS if successful;<br>

*             FLEXCAN_STATUS_TIMEOUT if fail to configure in configured timeout value.<br>

*/

FlexCAN_Ip_SetStopMode

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_SetStopMode(instance);

Description:

/**

*  @brief     Set the FlexCAN instance in STOP mode

*  @details   Set the FlexCAN instance in STOP mode, this will prevent instance to participate to

*  bus transactions and disable module clocks.

*  @param[in] instance A FlexCAN instance number

*  @return    FLEXCAN_STATUS_SUCCESS if successful;<br>

*             FLEXCAN_STATUS_TIMEOUT if fail to configure in configured timeout value.<br>

*

*/

FlexCAN_Ip_EnterFreezeMode

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_EnterFreezeMode(instance);

Description:

/**

*  @brief     Enter FlexCAN Module in Freeze Mode.

*  @details   This function will suspend bus activity of FlexCAN module and

*             set it to Freeze Mode to allow module configuration.

*  @param[in] instance A FlexCAN instance number

*  @return    FLEXCAN_STATUS_SUCCESS if successful;<br>

*             FLEXCAN_STATUS_TIMEOUT if fail to configure in the configured timeout value.<br>

*

*/

FlexCAN_Ip_ExitFreezeMode

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_ExitFreezeMode(instance);

Description:

/**

*  @brief     Exit FlexCAN Module from Freeze Mode.

*  @details   This function will allow FlexCAN module to participate to the BUS activity and

*             restore normal opertaion of the driver.

*  @param[in] instance A FlexCAN instance number

*  @note      This function should be called from FreezeMode.

*  @return    FLEXCAN_STATUS_SUCCESS if successful;<br>

*             FLEXCAN_STATUS_TIMEOUT if fail to configure in the configured timeout value.<br>

*

*/

FlexCAN_Ip_ConfigRxMb

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_ConfigRxMb(uint8 instance, uint8 mb_idx, const Flexcan_Ip_DataInfoType * rx_info, uint32 msg_id);

Description:

/**

*  @brief     FlexCAN receive message buffer field configuration

*  @details   This function will config receive parameters form Flexcan_Ip_DataInfoType and

*             the message Id, and can overwritte another MB status.

*  @param[in] instance A FlexCAN instance number

*  @param[in] mb_idx   Index of the message buffer

*  @param[in] rx_info  Data info

*  @param[in] msg_id   ID of the message to transmit

*  @return    FLEXCAN_STATUS_SUCCESS if successful;<br>

*             FLEXCAN_STATUS_BUFF_OUT_OF_RANGE if the index of a message buffer is invalid.

*

*/

FlexCAN_Ip_Init

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_Init(Flexcan_Ip_u8Instance, Flexcan_Ip_pState, Flexcan_Ip_pData);

Description:

/**

*  @brief Initializes the FlexCAN peripheral.

*  @details This function will config FlexCAN module and will leave the module in freeze mode.

*  @param[in] Flexcan_Ip_u8Instance A FlexCAN instance number

*  @param[in\out] Flexcan_Ip_pState Pointer to the FlexCAN driver state structure.

*  @param[in] Flexcan_Ip_pData      The FlexCAN platform configuration data

*  @return FLEXCAN_STATUS_SUCCESS if successfull;<br>

*          FLEXCAN_STATUS_ERROR if other error occurred;<br>

*          FLEXCAN_STATUS_BUFF_OUT_OF_RANGE if the index of a message buffer is invalid;<br>

*          FLEXCAN_STATUS_TIMEOUT if fail to configure in configured timeout value.<br>

*  @note In case of the enable option of Remote Request Store by setting corresponding bit for FLEXCAN_IP_REM_STORE_U32

*  in the ctrlOptions structure member of the Flexcan platform configuration data, will disable Automatic Response Request,

*  in this case is not allowed use of the function FlexCAN_Ip_ConfigRemoteResponseMb

*/

FlexCAN_Ip_Deinit

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_Deinit(instance);

Description:

/**

*  @brief     DeInitilize the FlexCAN instance driver

*  @details   This function will make future operataions of FlexCAN instance imposibile and will restore

*             it's state to default value as before initialization.

*  @param[in] instance A FlexCAN instance number

*  @return    FLEXCAN_STATUS_SUCCESS if successful;<br>

*             FLEXCAN_STATUS_TIMEOUT if fail to configure in configured timeout value.<br>

*

*/

FlexCAN_Ip_Receive

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_Receive(uint8 instance, uint8 mb_idx, Flexcan_Ip_MsgBuffType * data, boolean isPolling);

Description:

/**

*  @brief Receives a CAN frame using the specified message buffer.

*  @details This function receives a CAN frame using a configured message buffer. The function

*           returns immediately.

*  @param[in] instance  A FlexCAN instance number

*  @param[in] mb_idx    Index of the message buffer

*  @param[out] data     The FlexCAN receive message buffer data.

*  @param[in] isPolling If the message will be send using pooling(true) or interrupt(false).

*  @return FLEXCAN_STATUS_SUCCESS if successfull operation;<br>

*          FLEXCAN_STATUS_BUFF_OUT_OF_RANGE if the index of a message buffer is invalid;<br>

*          FLEXCAN_STATUS_BUSY if the message buffer is used for other operation.<br>

*

*/

FlexCAN_Ip_Send

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_Send(uint8 instance, uint8 mb_idx, const Flexcan_Ip_DataInfoType * tx_info, uint32 msg_id, const uint8 * mb_data);

Description:

/**

*  @brief     Sends a CAN frame using the specified message buffer.

*  @details   This function configure parameters form Flexcan_Ip_DataInfoType, ID and sends data as CAN frame using a message buffer.

*  @param[in] instance A FlexCAN instance number

*  @param[in] mb_idx   Index of the message buffer

*  @param[in] tx_info  Data info

*  @param[in] msg_id   ID of the message to transmit

*  @param[in] mb_data  Data Bytes of the FlexCAN message.

*  @return FLEXCAN_STATUS_BUFF_OUT_OF_RANGE if the index of a message buffer is invalid;<br>

*          FLEXCAN_STATUS_BUSY if the message buffer is used for other operation;<br>

*          FLEXCAN_STATUS_SUCCESS if successfull.<br>

*/

FlexCAN_Ip_MainFunctionWrite

Prototype:

void FlexCAN_Ip_MainFunctionWrite(uint8 instance, uint8 mb_idx);

Description:

/**

*  @brief     Check a Transmission event.

*  @details   This function will check a specific MB have been sent of FlexCAN module and

*             if was sent will reset the status of Mb and clear the status flag.

*  @param[in] instance A FlexCAN instance number

*  @param[in] mb_idx message buffer number

*

*/

FlexCAN_Ip_ClearBuffStatusFlag

Prototype:

boolean FlexCAN_Ip_GetBuffStatusFlag(uint8 instance, uint8 msgBuffIdx);

Description:

/**

*  @brief     Clear Message Buffer Status Flag

*  @details   This function will clear the status of the message buffer

*  @param[in] instance   A FlexCAN instance number

*  @param[in] msgBuffIdx Index of the message buffer

*

*/

FlexCAN_Ip_EnableInterrupts

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_EnableInterrupts(u8Instance);

Description:

/**

* @brief Enable all interrupts configured.

* @details Enable all interrupts configured.

* @param[in]   u8Instance    A FlexCAN instance number

* @return    FLEXCAN_STATUS_SUCCESS if successful<br>

*            FLEXCAN_STATUS_ERROR if fail to set<br>

*/

FlexCAN_Ip_ConfigRxFifo

Prototype:

FlexCAN_Ip_ConfigRxFifo(instance, id_format, id_filter_table)

Description:

/*!

* @brief FlexCAN Enhanced Rx FIFO field configuration

*

* @note The number of elements in the ID filter table is defined by the

*       following types:

*       - Type 1:ID filter element with filter + mask scheme

*       - Type 2: ID filter element with range scheme

*       - Type 3: ID filter element with 2-filter scheme

*       The user must provide the exact number of elements in order to avoid

*       any misconfiguration.

*       This function should be called from StopMode or FreezeMode.

*       Each element in the ID filter table specifies an ID to be used as

*       acceptance criteria for the Enhanced Rx FIFO as follows:

*       - for type 1: In the standard frame format, bits 26 to 16 of the ID

*         are used for frame identification and bits 10 to 0 are used for ID mask.

*         In the extended frame format, bits 28 to 0 are used for frame identification and

*         bits 28 to 0 of next work are used for ID mask.

*       - for type 2: In the standard frame format, bits 26 to 16 and bits 10 to 0 of the ID

*         are used for frame identification. The filter scheme is based on range.

*         In the extended frame format, bits 28 to 0 and bits 28 to 0 of next work

*         are used for frame identification. The filter scheme is based on range.

*       - for type 3: In the standard frame format, bits 26 to 16 and bits 10 to 0 of the ID

*         are used for frame identification.

*         In the extended frame format, bits 28 to 0 and bits 28 to 0 of next work

*         are used for frame identification.

* @param   instance           A FlexCAN instance number

* @param   id_filter_table    The ID filter table elements which contain Enhanced Rx FIFO

*                             filter type, RTR bit, IDE bit, and Rx message ID

*

* @return      FLEXCAN_STATUS_SUCCESS if successful;<br>

*              FLEXCAN_STATUS_ERROR if fail to set;<br>

*              FLEXCAN_STATUS_TIMEOUT if fail to configure in configured timeout value.<br>

*/

FlexCAN_Ip_RxFifo

Prototype:

Flexcan_Ip_StatusType FlexCAN_Ip_RxFifo(uint8 instance, Flexcan_Ip_MsgBuffType * data)

Description:

/*!

* @brief Receives a CAN frame using the message FIFO, in a blocking manner.

*

* This function receives a CAN frame using the Rx FIFO or

* Enhanced Rx FIFO (if available and enabled). If using Enhanced Rx FIFO, the size of

* the data array will be considered the same as the configured FIFO watermark.

* The function blocks until either a frame was received, or the specified timeout expired.

* FlexCAN_Ip_RxFifoBlocking/FlexCAN_Ip_RxFifo must not be called in callback invocation

* while FlexCAN_Ip_RxFifoBlocking is running to avoid unexpected behaviour.

*

* @param   instance    A FlexCAN instance number

* @param   data        The FlexCAN receive message buffer data.

* @param   timeout     A timeout for the transfer in milliseconds.

* @return  FLEXCAN_STATUS_SUCCESS if successful;

*          FLEXCAN_STATUS_BUSY if a resource is busy;

*          FLEXCAN_STATUS_TIMEOUT if the timeout is reached;

*          FLEXCAN_STATUS_ERROR if other error occurred

*/

Interrupt Controller (IntCtrl)

Below is an illustration of the Interrupt Controller component provided interfaces:

IntCtrl_Ip_Init

Prototype:

IntCtrl_Ip_StatusType IntCtrl_Ip_Init(const IntCtrl_Ip_CtrlConfigType *pIntCtrlCtrlConfig);

Description:

/**

* @brief         Initializes the configured interrupts at interrupt controller level.

*

* @details       This function is non-reentrant and initializes the interrupts.

*

* @param[in]     pIntCtrlCtrlConfig: pointer to configuration structure for interrupts.

* @return        IntCtrl_Ip_StatusType: error code.

*

* @api

*

* */

IntCtrl_Ip_InstallHandler

Prototype:

void IntCtrl_Ip_InstallHandler(IRQn_Type eIrqNumber,

const IntCtrl_Ip_IrqHandlerType pfNewHandler,

IntCtrl_Ip_IrqHandlerType* const pfOldHandler);

Description:

/**

* @brief         Installs a handler for an IRQ.

*

* @details       This function is non-reentrant; it installs an new ISR for an interrupt line.

* @note          This function works only when the interrupt vector table resides in RAM.

*

* @param[in]     eIrqNumber: interrupt number.

* @param[in]     pfNewHandler: function pointer for the new handler.

* @param[out]    pfOldHandler: stores the address of the old interrupt handler.

* @return        void.

*

* @api

*

* */

IntCtrl_Ip_EnableIrq

Prototype:

void IntCtrl_Ip_EnableIrq(IRQn_Type eIrqNumber);

Description:

/**

* @brief         Enables an interrupt request.

*

* @details       This function is non-reentrant; it enables the interrupt request at

*                interrupt controller level.

*

* @param[in]     eIrqNumber: interrupt number to be enabled.

* @return        void.

*

* @api

*

* */

IntCtrl_Ip_DisableIrq

Prototype:

void IntCtrl_Ip_DisableIrq(IRQn_Type eIrqNumber);

Description:

/**

* @brief         Disables an interrupt request.

*

* @details       This function is non-reentrant; it disables the interrupt request at

*                interrupt controller level.

*

* @param[in]     eIrqNumber: interrupt number to be disabled.

* @return        void.

*

* @api

*

* */

IntCtrl_Ip_SetPriority

Prototype:

void IntCtrl_Ip_SetPriority(IRQn_Type eIrqNumber, uint8 u8Priority);

Description:

/**

* @brief         Sets the priority for an interrupt request.

*

* @details       This function is non-reentrant; it sets the priority for the

*                interrupt request.

*

* @param[in]     eIrqNumber: interrupt number for which the priority is set.

* @param[in]     u8Priority: the priority to be set.

* @return        void.

*

* @api

*

* */

IntCtrl_Ip_GetPriority

Prototype:

uint8 IntCtrl_Ip_GetPriority(IRQn_Type eIrqNumber);

Description:

/**

* @brief         Gets the priority for an interrupt request.

*

* @details       This function is non-reentrant; it retrieves the priority for the

*                interrupt request.

*

* @param[in]     eIrqNumber: interrupt number for which the priority is set.

* @return        uint8: the priority of the interrupt.

*

* @api

*

* */

IntCtrl_Ip_ClearPending

Prototype:

void IntCtrl_Ip_ClearPending(IRQn_Type eIrqNumber);

Description:

/**

* @brief         Clears the pending flag for an interrupt request.

*

* @details       This function is reentrant; it clears the pending flag for the

*                interrupt request.

*

* @param[in]     eIrqNumber: interrupt number for which the pending flag is cleared.

* @return        void.

*

* @api

*

* */

IntCtrl_Ip_SetPending

Prototype:

void IntCtrl_Ip_SetPending(IRQn_Type eIrqNumber);

Description:

/**

* @brief         Sets the pending flag for an interrupt request.

*

* @details       This function is reentrant; it sets the pending flag for the

*                interrupt request.

*

* @param[in]     eIrqNumber: interrupt number for which the pending flag is set.

* @return        void.

*

* @api

*

* */

IntCtrl_Ip_GetPending

Prototype:

boolean IntCtrl_Ip_GetPending(IRQn_Type eIrqNumber);

Description:

/**

* @brief         Gets the pending flag for an interrupt request.

*

* @details       This function is reentrant; it retrieves the pending flag for the

*                interrupt request.

*

* @param[in]     eIrqNumber: interrupt number for which the pending flag is returned.

* @return        boolean: TRUE - pending flag set, FALSE - pending flag cleared.

*

* @api

*

* */

IntCtrl_Ip_GetActive

Prototype:

boolean IntCtrl_Ip_GetActive(IRQn_Type eIrqNumber);

Description:

/**

* @brief         Gets the active flag for an interrupt request.

*

* @details       This function is reentrant; it retrieves the active flag for the

*                interrupt request.

*

* @param[in]     eIrqNumber: interrupt number for which the active flag is returned.

* @return        boolean: TRUE - active flag set, FALSE - active flag cleared.

*

* @api

*

* */

Low-Power SPI (LPSPI)

Below is an illustration of the LPSPI component provided interfaces:

Lpspi_Ip_SyncTransmit

Prototype:

Lpspi_Ip_StatusType Lpspi_Ip_SyncTransmit(const Lpspi_Ip_ExternalDeviceType *ExternalDevice,

const uint8 *TxBuffer,

uint8 *RxBuffer,

uint16 Length,

uint32 TimeOut);

Description:

/**

* @brief            LPSPI synchronous transmission.

* @details          This function initializes a synchronous transfer using the bus parameters provided

*                   by external device.

*

* @param[in]        ExternalDevice - pointer to the external device where data is transmitted.

* @param[in]        TxBuffer - pointer to transmit buffer.

* @param[in,out]    RxBuffer - pointer to receive buffer.

* @param[in]        Length - number of bytes to be sent.

* @param[in]        TimeOut - duration for sending one frame.

*

* @return           LPSPI_IP_STATUS_SUCCESS: Transmission command has been accepted.

*                   LPSPI_IP_FIFO_ERROR: Overflow or underflow error occurred.

*                   LPSPI_IP_STATUS_FAIL: Transmission command has not been accepted.

*                   LPSPI_IP_TIMEOUT: Timeout error occurred.

*

* @implements Lpspi_Ip_SyncTransmit_Activity

*/

Lpspi_Ip_AsyncTransmit

Prototype:

Lpspi_Ip_StatusType Lpspi_Ip_AsyncTransmit(const Lpspi_Ip_ExternalDeviceType *ExternalDevice,

const uint8 *TxBuffer,

uint8 *RxBuffer,

uint16 Length,

Lpspi_Ip_CallbackType EndCallback);

Description:

/**

* @brief            LPSPI asynchronous transmission.

* @details          This function initializes an asynchronous transfer using the bus parameters provided

*                   by external device. After Lpspi_Ip_Init function is called, LPSPI_IP_POLLING

*                   mode is set as default to change the default mode Lpspi_Ip_UpdateTransferMode should be called.

*

* @param[in]        ExternalDevice - pointer to the external device where data is transmitted

* @param[in]        TxBuffer - pointer to transmit buffer.

* @param[in,out]    RxBuffer - pointer to receive buffer.

* @param[in]        Length - number of bytes to be sent.

* @param[in]        EndCallback - callback function is called at the end of transfer.

*

* @return           LPSPI_IP_STATUS_SUCCESS: Transmission command has been accepted.

*                   LPSPI_IP_STATUS_FAIL: Transmission command has not been accepted.

* @implements Lpspi_Ip_AsyncTransmit_Activity

*/

Lpspi_Ip_GetStatus

Prototype:

Lpspi_Ip_HwStatusType Lpspi_Ip_GetStatus(uint8 Instance);

Description:

/**

* @brief            Get status of HW unit.

* @details          This function returns the status of the specified SPI Hardware microcontroller peripheral.

*

* @param[in]        Instance - SPI peripheral instance number.

*

* @return           Lpspi_Ip_HwStatusType

* @implements Lpspi_Ip_GetStatus_Activity

*/

Lpspi_Ip_ManageBuffers

Prototype:

void Lpspi_Ip_ManageBuffers(uint8 Instance);

Description:

/**

* @brief            Process transfer in POLLING mode.

* @details          This function shall polls the SPI interrupts linked to SPI peripheral instance allocated to

*                   the transmission of data to enable the evolution of transmission state machine.

*

* @param[in]        Instance - SPI peripheral instance number.

* @implements Lpspi_Ip_ManageBuffers_Activity

*/

Lpspi_Ip_UpdateTransferParam

Prototype:

Lpspi_Ip_StatusType Lpspi_Ip_UpdateTransferParam(const Lpspi_Ip_ExternalDeviceType *ExternalDevice,

const Lpspi_Ip_TransferAdjustmentType *Param);

Description:

/**

* @brief            Adjust transfer based on external conditions

* @details          Use parameters to improve transfer

*

* @param[in]        ExternalDevice - External device where data is transmitted

* @param[in]        Param - Parameter with adjustments

*

* @return           LPSPI_IP_STATUS_SUCCESS: Setting command has been accepted.

*                   LPSPI_IP_STATUS_FAIL: Setting command has not been accepted.

*/

Lpspi_Ip_UpdateFrameSize

Prototype:

Lpspi_Ip_StatusType Lpspi_Ip_UpdateFrameSize(const Lpspi_Ip_ExternalDeviceType *ExternalDevice,

uint16 FrameSize);

Description:

/**

* @brief            LPSPI change frame size.

* @details          This function updates frame size of specific external device configuration for next transfers.

*

* @param[in]        ExternalDevice - pointer to the external device configuration.

* @param[in]        FrameSize - Frame size.

*

* @return           LPSPI_IP_STATUS_SUCCESS: Setting command has been accepted.

*                   LPSPI_IP_STATUS_FAIL: Setting command has not been accepted.

* @implements Lpspi_Ip_UpdateFrameSize_Activity

*/

Lpspi_Ip_UpdateLsb

Prototype:

Lpspi_Ip_StatusType Lpspi_Ip_UpdateLsb(const Lpspi_Ip_ExternalDeviceType *ExternalDevice,

boolean Lsb);

Description:

/**

* @brief            LPSPI change bit order.

* @details          This function updates bits order LSB or MSB of specific external device configuration for next transfer.

*

* @param[in]        ExternalDevice - pointer to the external device configuration.

* @param[in]        Lsb - Data is transferred LSB first or not.

*

* @return           LPSPI_IP_STATUS_SUCCESS: Setting command has been accepted.

*                   LPSPI_IP_STATUS_FAIL: Setting command has not been accepted.

* @implements Lpspi_Ip_UpdateLsb_Activity

*/

Lpspi_Ip_UpdateByteSwap

Prototype:

Lpspi_Ip_StatusType Lpspi_Ip_UpdateByteSwap(const Lpspi_Ip_ExternalDeviceType *ExternalDevice,

boolean ByteSwap);

Description:

/**

* @brief            LPSPI change byte swap.

* @details          This function updates to enable byte swap on each 32-bit word when transmitting and receiving

data.

*

* @param[in]        ExternalDevice - pointer to the external device configuration.

* @param[in]        ByteSwap - Data is transferred with byte swap feature or not.

*

* @return           LPSPI_IP_STATUS_SUCCESS: Setting command has been accepted.

*                   LPSPI_IP_STATUS_FAIL: Setting command has not been accepted.

* @implements Lpspi_Ip_UpdateByteSwap_Activity

*/

Lpspi_Ip_UpdateDefaultTransmitData

Prototype:

Lpspi_Ip_StatusType Lpspi_Ip_UpdateDefaultTransmitData(const Lpspi_Ip_ExternalDeviceType *ExternalDevice,

uint32 DefaultData);

Description:

/**

* @brief            LPSPI change default transmit data.

* @details          This function updates default transmit data of specific external device configuration for next transfer.

*

* @param[in]        ExternalDevice - pointer to the external device configuration.

* @param[in]        DefaultData - New default transmit data.

*

* @return           LPSPI_IP_STATUS_SUCCESS: Setting command has been accepted.

*                   LPSPI_IP_STATUS_FAIL: Setting command has not been accepted.

* @implements Lpspi_Ip_UpdateDefaultTransmitData_Activity

*/

Lpspi_Ip_UpdateTransferMode

Prototype:

Lpspi_Ip_StatusType Lpspi_Ip_UpdateTransferMode(uint8 Instance,

Lpspi_Ip_ModeType Mode);

Description:

/**

* @brief            LPSPI change transfer mode.

* @details          This function updates the asynchronous mechanism mode for the specified SPI Hardware microcontroller peripheral.

*

* @param[in]        Instance - SPI peripheral instance number.

* @param[in]        Mode - new mode (interrupt or polling).

*

* @return           LPSPI_IP_STATUS_SUCCESS: Setting command has been accepted.

*                   LPSPI_IP_STATUS_FAIL: Setting command has not been accepted.

* @implements Lpspi_Ip_UpdateTransferMode_Activity

*/

Lpspi_Ip_Cancel

Prototype:

void Lpspi_Ip_Cancel(uint8 Instance);

Description:

/**

* @brief            LPSPI cancel current asynchronous transmission.

* @details          This function cancels an asynchronous transmission in progress for the specified SPI Hardware microcontroller peripheral.

*

* @param[in]        Instance - SPI peripheral instance number.

* @implements Lpspi_Ip_Cancel_Activity

*/

System Integration Unit Lite2 (SIUL2)

Below is an illustration of the provided interfaces for the SIUL2 component:

Siul2_Port_Ip_SetPullSel

Prototype:

void Siul2_Port_Ip_SetPullSel(Siul2_Port_Ip_PortType * const base,

uint16 pin,

Siul2_Port_Ip_PortPullConfig pullConfig);

Description:

/*!

* @brief Configures the internal resistor.

*

* This function configures the internal resistor.

*

* @param[in] base Port base pointer (PORTA, PORTB, PORTA_AE, etc.)

* @param[in] pin Port pin number

* @param[in] pullConfig The pull configuration

*/

Siul2_Port_Ip_SetOuptutBuffer

Prototype:

void Siul2_Port_Ip_SetOutputBuffer(Siul2_Port_Ip_PortType * const base,

uint16 pin,

boolean enable,

Siul2_Port_Ip_PortMux mux);

Description:

/*!

* @brief Configures the output buffer and output signal.

*

* This function configures the output buffer for the pin

* and the path for output signal from module to pin

*

* @param[in] base Port base pointer (PORTA, PORTB, PORTA_AE, etc.)

* @param[in] pin Port pin number

* @param[in] enable Enable output buffer

* @param[in] mux Pin muxing slot selection

*/

Siul2_Port_Ip_SetInputBuffer

Prototype:

void Siul2_Port_Ip_SetInputBuffer(Siul2_Port_Ip_PortType * const base,

uint16 pin,

boolean enable,

uint32 inputMuxReg,

Siul2_Port_Ip_PortInputMux inputMux);

Description:

/*!

* @brief Configures the input buffer and input signal.

*

* This function configures the input buffer for the pin

* and the path for input signal from pin to module

*

* @param[in] base Port base pointer (PORTA, PORTB, PORTA_AE, etc.),

*            NULL if disabling inputMux only

* @param[in] pin Port pin number

* @param[in] enable Enable input buffer

* @param[in] inputMuxReg Pin muxing register slot selection

* @param[in] inputMux Pin muxing slot selection

*

* @note: There are some pins controlled by both SIUL2_0, SIUL2_1, SIUL2_3, SIUL2_4 and SIUL2_5 instances

* In order to configure correctly and be consistent with other platforms, the

* inputMuxReg parameter of SIUL2_3 instance must be added 512 units.

* For example: The actual inputMuxReg is 10 then the value there must be (10 + 512)

*/

Siul2_Port_Ip_Init

Prototype:

Siul2_Port_Ip_PortStatusType Siul2_Port_Ip_Init(uint32 pinCount,

const Siul2_Port_Ip_PinSettingsConfig config[]);

Description:

/*!

* @brief Initializes the pins with the given configuration structure

*

* This function configures the pins with the options provided in the

* provided structure.

*

* @param[in] pinCount The number of configured pins in structure

* @param[in] config The configuration structure

* @return The status of the operation

*/

Siul2_Port_Ip_SetPinDirection

Prototype:

void Siul2_Port_Ip_SetPinDirection(Siul2_Port_Ip_PortType * const base,

uint16 pin,

Siul2_Port_Ip_PortDirectionType direction);

Description:

/*!

* @brief Configures the pin with the values form the configuration structure

*

* This function configures the pin configuration with the values form the configuration structure

*

* @param[in] base Port base pointer

* @param[in] pin Port pin number

* @param[in] direction The direction of pin

* @return void

*/

Siul2_Port_Ip_RevertPinConfiguration

Prototype:

uint32 Siul2_Port_Ip_RevertPinConfiguration(const Siul2_Port_Ip_PortType * const base,

uint16 pin);

Description:

/*!

* @brief This function configures the pin configuration with the values from the configuration structure

*

* This function configures the pin configuration with the values from the configuration structure

*

* @param[in] base Port base pointer

* @param[in] pin Port pin number

* @return MSCR register value

*/

Siul2_Port_Ip_GetPinConfiguration

Prototype:

void Siul2_Port_Ip_GetPinConfiguration(const Siul2_Port_Ip_PortType * const base,

Siul2_Port_Ip_PinSettingsConfig * config,

uint16 pin);

Description:

/*!

* @brief This function shall return the value of the pin configuration register.

*

* This function shall return the value of the pin configuration register.

*

* @param[in] base Port base pointer

* @param[in] pin Port pin number

* @param[out] config->pointer to output configuration structure information

* @return MSCR register value

*/

Siul2_Dio_Ip_WritePin

Prototype:

void Siul2_Dio_Ip_WritePin(Siul2_Dio_Ip_GpioType * const base,

Siul2_Dio_Ip_PinsChannelType pin,

Siul2_Dio_Ip_PinsLevelType value);

Description:

/*!

* @brief Write a pin of a port with a given value

*

* This function writes the given pin from a port, with the given value

* ('0' represents LOW, '1' represents HIGH).

*

* @param base  GPIO base pointer (PTA, PTB, PTC, etc.)

* @param pin pin number to be written

* @param value pin value to be written

*        - 0: corresponding pin is set to LOW

*        - 1: corresponding pin is set to HIGH

*/

Siul2_Dio_Ip_SetGPDO

Prototype:

void Siul2_Dio_Ip_SetGPDO(const uint8 u8Siul2Instance,

const uint16 GPDOnum);

Description:

/*!

* @brief Write the GPDO register a pin of a port with a given value

*

* This function sets the GPDO register for a given Siul2 Instance and GPDO register number

*

* @param u8Siul2Instance  number of Siul2Instance

* @param GPDOnum  number of GPDO

*/

Siul2_Dio_Ip_ClearGPDO

Prototype:

void Siul2_Dio_Ip_ClearGPDO(const uint8 u8Siul2Instance,

const uint16 GPDOnum);

Description:

/**

* @brief            LPSPI cancel current asynchronous transmission.

* @details          This function cancels an asynchronous transmission in progress for the specified SPI Hardware microcontroller peripheral.

*

* @param[in]        Instance - SPI peripheral instance number.

* @implements Lpspi_Ip_Cancel_Activity

*/

Siul2_Dio_Ip_WritePins

Prototype:

void Siul2_Dio_Ip_WritePins(Siul2_Dio_Ip_GpioType * const base,

Siul2_Dio_Ip_PinsChannelType pins);

Description:

/*!

* @brief Write all pins of a port

*

* This function writes all pins configured as output with the values given in

* the parameter pins. '0' represents LOW, '1' represents HIGH.

*

* @param[in] base GPIO base pointer (PTA, PTB, PTC, etc.)

* @param[in] pins Pin mask to be written

*        - 0: corresponding pin is set to LOW

*        - 1: corresponding pin is set to HIGH

*/

Siul2_Dio_Ip_GetPinsOutput

Prototype:

Siul2_Dio_Ip_PinsChannelType Siul2_Dio_Ip_GetPinsOutput(const Siul2_Dio_Ip_GpioType * const base);

Description:

/*!

* @brief Get the current output from a port

*

* This function returns the current output that is written to a port. Only pins

* that are configured as output will have meaningful values.

*

* @param[in] base GPIO base pointer (PTA, PTB, PTC, etc.)

* @return GPIO outputs. Each bit represents one pin (LSB is pin 0, MSB is pin

* 31). For each bit:

*        - 0: corresponding pin is set to LOW

*        - 1: corresponding pin is set to HIGH

*/

Siul2_Dio_Ip_SetPins

Prototype:

void Siul2_Dio_Ip_SetPins(Siul2_Dio_Ip_GpioType * const base,

Siul2_Dio_Ip_PinsChannelType pins);

Description:

/*!

* @brief Write pins with 'Set' value

*

* This function configures output pins listed in parameter pins (bits that are

* '1') to have a value of 'set' (HIGH). Pins corresponding to '0' will be

* unaffected.

*

* @param[in] base GPIO base pointer (PTA, PTB, PTC, etc.)

* @param[in] pins Pin mask of bits to be set. Each bit represents one pin (LSB is

* pin 0, MSB is pin 31). For each bit:

*        - 0: corresponding pin is unaffected

*        - 1: corresponding pin is set to HIGH

*/

Siul2_Dio_Ip_ClearPins

Prototype:

void Siul2_Dio_Ip_ClearPins(Siul2_Dio_Ip_GpioType * const base,

Siul2_Dio_Ip_PinsChannelType pins);

Description:

/*!

* @brief Write pins to 'Clear' value

*

* This function configures output pins listed in parameter pins (bits that are

* '1') to have a 'cleared' value (LOW). Pins corresponding to '0' will be

* unaffected.

*

* @param[in] base GPIO base pointer (PTA, PTB, PTC, etc.)

* @param[in] pins Pin mask of bits to be cleared. Each bit represents one pin (LSB

* is pin 0, MSB is pin 31). For each bit:

*        - 0: corresponding pin is unaffected

*        - 1: corresponding pin is cleared(set to LOW)

*/

Siul2_Dio_Ip_TogglePins

Prototype:

void Siul2_Dio_Ip_TogglePins(Siul2_Dio_Ip_GpioType * const base,

Siul2_Dio_Ip_PinsChannelType pins);

Description:

/*!

* @brief Toggle pins value

*

* This function toggles output pins listed in parameter pins (bits that are

* '1'). Pins corresponding to '0' will be unaffected.

*

* @param[in] base GPIO base pointer (PTA, PTB, PTC, etc.)

* @param[in] pins Pin mask of bits to be toggled.  Each bit represents one pin (LSB

* is pin 0, MSB is pin 31). For each bit:

*        - 0: corresponding pin is unaffected

*        - 1: corresponding pin is toggled

*/

Siul2_Dio_Ip_ReadPins

Prototype:

Siul2_Dio_Ip_PinsChannelType Siul2_Dio_Ip_ReadPins(const Siul2_Dio_Ip_GpioType * const base);

Description:

/*!

* @brief Read input pins

*

* This function returns the current input values from a port. Only pins

* configured as input will have meaningful values.

*

* @param[in] base GPIO base pointer (PTA, PTB, PTC, etc.)

* @return GPIO inputs. Each bit represents one pin (LSB is pin 0, MSB is pin

* 31). For each bit:

*        - 0: corresponding pin is read as LOW

*        - 1: corresponding pin is read as HIGH

*/

Siul2_Dio_Ip_MaskedWritePins

Prototype:

void Siul2_Dio_Ip_MaskedWritePins(uint8 u8Siul2Instance,

uint8 u8PortId,

Siul2_Dio_Ip_PinsChannelType pins,

Siul2_Dio_Ip_PinsChannelType mask);

Description:

/*!

* @brief Write Port using MPGPDO

*

* This function writes the masked pins configured as output with the values given in the parameter pins.

* '0' represents LOW, '1' represents HIGH.

*

* @param[in] u8Siul2Instance: instance number

* @param[in] u8PortId: id of port need to be written (corresponds with index number of GPIO base (0-PTA, 1-PTB, etc))

* @param[in] pins: Pin mask of bits to be cleared. Each bit represents one pin (LSB is pin 0, MSB is pin 31). For each bit: 0: corresponding

*                  pin is unaffected. 1: corresponding pin is cleared(set to LOW).

* @param[in] mask: mask for the affected pins

* @return none

*/

Siul2_Dio_Ip_ReadPin

Prototype:

Siul2_Dio_Ip_PinsLevelType Siul2_Dio_Ip_ReadPin(const Siul2_Dio_Ip_GpioType * const base,

Siul2_Dio_Ip_PinsChannelType pin);

Description:

/*!

* @brief Read input pin

*

* This function returns the current  input value of the given pin from port. Only pin

* configured as input will have meaningful value.

*

* @param[in] base GPIO base pointer (PTA, PTB, PTC, etc.)

* @param[in] pin Pin index (0,1,2,3,..,15)

* @return GPIO input value for coressponding pin

*        - 0: corresponding pin is read as LOW

*        - 1: corresponding pin is read as HIGH

*/

Siul2_Icu_Ip_DeInit

Prototype:

Siul2_Icu_Ip_StatusType Siul2_Icu_Ip_DeInit(uint8 instance);

Description:

/**

* @brief      Driver function that de-initializes SIUL hardware channel.

* @details    This function:

*              - Restore to reset values SIUL2 registers used on init.

*

* @param[in]  instance - Instance number used

* @return     Siul2_Icu_Ip_StatusType - The status of DeInit

*/

Siul2_Icu_Ip_Initi

Prototype:

Siul2_Icu_Ip_StatusType Siul2_Icu_Ip_Init(uint8 instance, const Siul2_Icu_Ip_ConfigType* userConfig);

Description:

/**

* @brief      Driver function that de-initializes SIUL hardware channel.

* @details    This function:

*              - Restore to reset values SIUL2 registers used on init.

*

* @param[in]  instance - Instance number used

* @return     Siul2_Icu_Ip_StatusType - The status of DeInit

*/

Siul2_Icu_Ip_SetActivationCondition

Prototype:

void Siul2_Icu_Ip_SetActivationCondition(uint8 instance, uint8 hwChannel, Siul2_Icu_Ip_EdgeType edge);

Description:

/**

* @brief

* @details   This function enables the requested activation condition (rising, falling or both

*            edges) for corresponding SIUL2 channels.

*

* @param[in] instance Hardware instance of SIUL2 used.

* @param[in] hwChannel Hardware channel of SIUL2 used.

* @param[in] edge      Edge activation type used.

*/

Siul2_Icu_Ip_GetInputState

Prototype:

boolean Siul2_Icu_Ip_GetInputState(uint8 instance, uint8 hwChannel);

Description:

/**

* @brief ICU driver function that sets activation condition of SIUL2 channel.

*

* @param[in]      instance  Hardware instance of SIUL2 used.

* @param[in]      hwChannel Hardware channel of SIUL2 used.

* @return boolean Input state.

*/

Siul2_Icu_Ip_Enable Interrupt

Prototype:

void Siul2_Icu_Ip_EnableInterrupt(uint8 instance, uint8 hwChannel);

Description:

/**

* @brief      ICU driver function that enables the interrupt of SIUL2 channel.

* @details    This function enables SIUL2 Channel Interrupt.

*

* @param[in]  instance  Hardware instance of SIUL2 used.

* @param[in]  hwChannel Hardware channel of SIUL2 used.

* @return     void

*/

Siul2_Icu_Ip_DisableInterrupt

Prototype:

void Siul2_Icu_Ip_DisableInterrupt(uint8 instance, uint8 hwChannel);

Description:

/**

* @brief      ICU driver function that disables the interrupt of SIUL2 channel.

* @details    This function disables SIUL2 Channel Interrupt.

*

* @param[in]  instance  Hardware instance of SIUL2 used.

* @param[in]  hwChannel Hardware channel of SIUL2 used.

* @return     void

*/

Siul2_Icu_Ip_SetClockMode

Prototype:

void Siul2_Icu_Ip_SetClockMode(uint8 instance, Siul2_Icu_Ip_ClockModeType mode);

Description:

/**

* @brief      Icu driver function used to set the global prescaler of a SIUL2 module.

* @details    This function:

*             - Sets IFCPR register with a prescaler value

*

* @param[in]  instance  Hardware instance of SIUL2 used.

* @param[in]  mode Global prescaler for the SIUL2 module.

* @return     void

*/

Siul2_Icu_Ip_EnableNotification

Prototype:

void Siul2_Icu_Ip_EnableNotification(uint8 instance, uint8 hwChannel);

Description:

/**

* @brief      Driver function Enable Notification for timestamp.

*

* @param[in]  instance  Hardware instance of FTM used.

* @param[in]  hwChannel Hardware channel of FTM used.

* @return     void

*/

Siul2_Icu_Ip_DisableNotification

Prototype:

void Siul2_Icu_Ip_DisableNotification(uint8 instance, uint8 hwChannel);

Description:

/**

* @brief      Driver function Disable Notification for timestamp.

*

* @param[in]  instance  Hardware instance of FTM used.

* @param[in]  hwChannel Hardware channel of FTM used.

* @return     void

*/

ADC Successive Approximation Register (SAR)

Below is an illustration of the provided interfaces for the ADC SAR component:

Adc_Sar_Ip_GetDataAddress

Prototype:

uint32 Adc_Sar_Ip_GetDataAddress(uint32 u32Instance,

uint32 u32ChannelIndex);

Description:

/*!

* @brief Return the address of the specified data register

*

* This function returns the address of the specified data register

*

* @param[in] u32Instance - ADC instance number

* @param[in] u32ChannelIndex - adc channel of the Hw unit

* @return status:

* - value of the address of the data for the specified channel

*/

Adc_Sar_Ip_ChainConfig

Prototype:

void Adc_Sar_Ip_ChainConfig(const uint32 u32Instance,

const Adc_Sar_Ip_ChansIdxMaskType * const pChansIdxMask,

const Adc_Sar_Ip_ConvChainType pChainType);

Description:

/*!

* @brief Configures the converter chains the given configuration structure

*

* This function configures the ADC Normal and Injected Chains with the options

* provided in the structure.

*

* @param[in] u32Instance - ADC instance number

* @param[in] pChansIdxMask - channel configuration structure

* @param[in] pChainType - conversion chain (Normal or Injected)

* @return void

*/

Adc_Sar_Ip_EnableChannel

Prototype:

void Adc_Sar_Ip_EnableChannel(const uint32 u32Instance,

const Adc_Sar_Ip_ConvChainType pChainType,

const uint32 u32ChnIdx);

Description:

/*!

* @brief Enable a channel

*

* This function enables a channel in a specified conversion chain

*

* @param[in] u32Instance - ADC instance number

* @param[in] pChainType - conversion chain (Normal or Injected)

* @param[in] u32ChnIdx - channel index(channel number in the RM)

* @return void

*/

Adc_Sar_Ip_DisableChannel

Prototype:

void Adc_Sar_Ip_DisableChannel(const uint32 u32Instance,

const Adc_Sar_Ip_ConvChainType pChainType,

const uint32 u32ChnIdx);

Description:

/*!

* @brief Disable a channel

*

* This function disables a channel in a specified conversion chain

*

* @param[in] u32Instance - ADC instance number

* @param[in] pChainType - conversion chain (Normal or Injected)

* @param[in] u32ChnIdx - channel index(channel number in the RM)

* @return void

*/

Adc_Sar_Ip_SetResolution

Prototype:

void Adc_Sar_Ip_SetResolution(const uint32 u32Instance,

const Adc_Sar_Ip_Resolution eResolution);

Description:

/*!

* @brief Set conversion resolution

*

* This function sets the conversion resolution (number of bits per conversion data)

*

* @param[in] u32Instance - ADC instance number

* @param[in] eResolution - conversion resolution

* @return void

*/

Adc_Sar_Ip_StartConversion

Prototype:

void Adc_Sar_Ip_StartConversion(const uint32 u32Instance,

const Adc_Sar_Ip_ConvChainType pChainType);

Description:

/*!

* @brief Start conversion

*

* This function starts a conversion channel (Normal or Injected)

*

* @param[in] u32Instance - ADC instance number

* @param[in] pChainType - conversion chain (Normal or Injected)

* @return void

*/

Adc_Sar_Ip_GetStatusFlags

Prototype:

uint32 Adc_Sar_Ip_GetStatusFlags(const uint32 u32Instance);

Description:

/*!

* @brief Get the status flags

*

* This function returns the status flags of the ADC.

*

* @param[in] u32Instance - ADC instance number

* @return the status flag bit-mask

*/

Adc_Sar_Ip_ClearStatusFlags

Prototype:

void Adc_Sar_Ip_ClearStatusFlags(const uint32 u32Instance,

const uint32 u32Mask);

Description:

/*!

* @brief Clear the status flags

*

* This function clears the status flags of the ADC.

*

* @param[in] u32Instance - ADC instance number

* @param[in] u32Mask - bit-mask of flags to clear

* @return void

*/

Adc_Sar_Ip_GetConvDataToArray

Prototype:

uint32 Adc_Sar_Ip_GetConvDataToArray(const uint32 u32Instance,

const Adc_Sar_Ip_ConvChainType pChainType,

const uint32 u32Length,

uint16 * const pResults);

Description:

/*!

* @brief Get conversion results for a conversion chain

*

* This function gets the conversion results for the selected Conversion Chain.

*

* @param[in] u32Instance - ADC instance numberAdc_Sar_Ip_DoCalibration

* @param[in] pChainType - conversion chain (Normal, Injected or CTU)

* @param[in] u32Length - the length of the buffer

* @param[out] pResults - the output buffer

* @return the number of values written in the buffer (max length)

*/

Adc_Sar_Ip_GetConvResultsToArray

Prototype:

uint32 Adc_Sar_Ip_GetConvResultsToArray(const uint32 u32Instance,

const Adc_Sar_Ip_ConvChainType pChainType,

const uint32 u32Length,

Adc_Sar_Ip_ChanResultType * const pResults);

Description:

/*!

* @brief Get conversion results for a conversion chain with extended information

*

* This function gets the conversion results for the selected Conversion Chain, with

* extended information about each conversion result (channel index, valid an overwritten

* properties and conversion data). This function should be used in case of configurations

* with overlapping channel lists in different chains, resulting in overwrite of conversion

* data when a higher priority chain is executed before all data was read.

*

* @param[in] u32Instance - ADC instance number

* @param[in] pChainType - conversion chain (Normal, Injected or CTU)

* @param[in] u32Length - the length of the buffer

* @param[out] pResults - the output buffer

* @return the number of values written in the buffer (max length)

*/

Adc_Sar_Ip_GetConvData

Prototype:

uint16 Adc_Sar_Ip_GetConvData(const uint32 u32Instance,

const uint32 u32ChnIdx);

Description:

/*!

* @brief Return the result of the conversion

*

* This function returns the result of the conversion for a single channel

*

* @param [in] u32Instance - ADC instance number

* @param [in] u32ChnIdx - channel index(channel number in the RM)

* @return conversion data

*/

Adc_Sar_Ip_GetConvResult

Prototype:

void Adc_Sar_Ip_GetConvResult(const uint32 u32Instance,

const uint32  u32ChnIdx,

const Adc_Sar_Ip_ConvChainType pChainType,

Adc_Sar_Ip_ChanResultType * const pResult);

Description:

/*!

* @brief Return the result and the status of the conversion

*

* This function returns the result and the status of the conversion

* for a single channel

*

* @param [in] u32Instance - ADC instance number

* @param [in] u32ChnIdx - channel index(channel number in the RM)

* @param [in] pChainType - conversion chain (Normal, Injected or CTU)

* @param [out] pResult - pointer to the buffer where the result will be written

* @return void

*/

Adc_Sar_Ip_DoCalibration

Prototype:

Adc_Sar_Ip_StatusType Adc_Sar_Ip_DoCalibration(const uint32 u32Instance);

Description:

/*!

* @brief Perform Calibration of the ADC

*

* This function performs a calibration of the ADC. The maximum input clock

* frequency for the ADC is 80 MHz, checked with assertions if DEV_ASSERT is

* enabled. After calibration, the ADC is left in Powerup state (PWDN bit is clear).

* Note: If asynchronous calibration mode is enabled and a calibration is started,

* other APIs should not be called until the calibration is complete.

*

* @param[in] u32Instance - ADC instance number

* @return the calibration result

*  - ADC_SAR_IP_STATUS_SUCCESS: calibration successful

*  - ADC_SAR_IP_STATUS_TIMEOUT: calibration step timed out

*  - ADC_SAR_IP_STATUS_ERROR: calibration failed

*  - ADC_SAR_IP_STATUS_CAL_ONGOING: calibration is ongoing (if async mode is active)

*/

Adc_Sar_Ip_Powerup

Prototype:

Adc_Sar_Ip_StatusType Adc_Sar_Ip_Powerup(const uint32 u32Instance);

Description:

/*!

* @brief Power up the ADC

*

* This function enables the ADC (disables the Power Down feature).

*

* @param[in] u32Instance - ADC instance number

* @return status:

*  - ADC_SAR_IP_STATUS_SUCCESS: power up successful

*  - ADC_SAR_IP_STATUS_TIMEOUT: power up failed

*/

Adc_Sar_Ip_Powerdown

Prototype:

Adc_Sar_Ip_StatusType Adc_Sar_Ip_Powerdown(const uint32 u32Instance);

Description:

/*!

* @brief Power down the ADC

*

* This function disables the ADC (enables the Power Down feature).

*

* @param[in] u32Instance - ADC instance number

* @return status:

*  - ADC_SAR_IP_STATUS_SUCCESS: power down successful

*  - ADC_SAR_IP_STATUS_TIMEOUT: power down failed

*/

Adc_Sar_Ip_EnableNotifications

Prototype:

void Adc_Sar_Ip_EnableNotifications(const uint32 u32Instance,

const uint32 u32NotificationMask);

Description:

/*!

* @brief Enable ADC interrupts

*

* This function enables ADC interrupts.

*

* @param[in] u32Instance - ADC instance number

* @param[in] u32NotificationMask - mask of interrupts to enable (of status flags)

* @return void

*/

Adc_Sar_Ip_DisableNotifications

Prototype:

void Adc_Sar_Ip_DisableNotifications(const uint32 u32Instance,

const uint32 u32NotificationMask);

Description:

/*!

* @brief Disable ADC interrupts

*

* This function disables ADC interrupts.

*

* @param[in] u32Instance - ADC instance number

* @param[in] u32NotificationMask - mask of interrupts to disable (of status flags)

* @return void

*/

Adc_Sar_Ip_EnableChannelNotifications

Prototype:

void Adc_Sar_Ip_EnableChannelNotifications(const uint32 u32Instance,

const uint32 u32ChnIdx,

const uint32 u32Mask);

Description:

/*!

* @brief Enable ADC interrupt for a channel

*

* This function enables interrupt generation on End of Conversion and watchdog

* events for a single channel.

* The mask parameter can be set using the defines provided in the driver header

* file that have the pattern ADC_SAR_IP_CHAN_NOTIF_... e.g. ADC_SAR_IP_CHAN_NOTIF_EOC.

*

* @param[in] u32Instance - ADC instance number

* @param[in] u32ChnIdx - channel index(channel number in the RM)

* @param[in] u32Mask - mask selecting targeted events

* @return void

*/

Adc_Sar_Ip_DisableChannelNotifications

Prototype:

void Adc_Sar_Ip_DisableChannelNotifications(const uint32 u32Instance,

const uint32 u32ChnIdx,

const uint32 u32Mask);

Description:

/*!

* @brief Disable ADC interrupt for a channel

*

* This function disables interrupt generation on End of Conversion and watchdog

* events for a single channel.

* The mask parameter can be set using the defines provided in the driver header

* file that have the pattern ADC_SAR_IP_CHAN_NOTIF_... e.g. ADC_SAR_IP_CHAN_NOTIF_EOC.

*

* @param[in] u32Instance - ADC instance number

* @param[in] u32ChnIdx - channel index(channel number in the RM)

* @param[in] u32Mask - mask selecting targeted events

* @return void

*/

Adc_Sar_Ip_SetClockMode

Prototype:

Adc_Sar_Ip_StatusType Adc_Sar_Ip_SetClockMode(const uint32 u32Instance,

const Adc_Sar_Ip_ClockConfigType * const pConfig);

Description:

/*!

* @brief Set the ADC clocks

*

* This function initializes the ADC clock configuration.

*

* @param[in] u32Instance - ADC instance number

* @param[in] pConfig - the clock configuration

* @return status:

*  - ADC_SAR_IP_STATUS_SUCCESS: set successful

*  - ADC_SAR_IP_STATUS_TIMEOUT: power up or down sequence timed out

*/

Adc_Sar_Ip_SetSampleTimes

Prototype:

void Adc_Sar_Ip_SetSampleTimes(const uint32 u32Instance,

const uint8 * const aSampleTimes);

Description:

/*!

* @brief Set the sample times

*

* This function sets the sample times for each channnel group.

* Note: aSampleTimes must contain the sample times in order of the channel

* groups, e.g. sample time for the channel group 1 must be on the first position.

*

* @param[in] u32Instance - ADC instance number

* @param[in] aSampleTimes - array with size ADC_SAR_IP_NUM_GROUP_CHAN containing sample times for each channel group

* @return void

*/

Adc_Sar_Ip_SetAveraging

Prototype:

void Adc_Sar_Ip_SetAveraging(const uint32 u32Instance,

const boolean bAvgEn,

const Adc_Sar_Ip_AvgSelectType eAvgSel);

Description:

/*!

* @brief Configure averaging

*

* This function enables averaging and selects the number of conversions to average.

* The mask parameter should be set using the Adc_Sar_Ip_AvgSelectType enum elements

* that have the pattern ADC_SAR_IP_AVG_... e.g. ADC_SAR_IP_AVG_4_CONV.

*

* @param[in] u32Instance - ADC instance number

* @param[in] bAvgEn - enable or disable averaging

* @param[in] eAvgSel - selects number of conversions to average

* @return void

*/

Adc_Sar_Ip_AbortConversion

Prototype:

void Adc_Sar_Ip_AbortConversion(const uint32 u32Instance);

Description:

/*!

* @brief Abort ongoing conversion

*

* This function aborts an ongoing conversion.

*

* @param[in] u32Instance - ADC instance number

* @return void

*/

Adc_Sar_Ip_AbortChain

Prototype:

Adc_Sar_Ip_StatusType Adc_Sar_Ip_AbortChain(const uint32 u32Instance,

const boolean bBlocking,

const boolean bAllowRestart);

Description:

/*!

* @brief Abort ongoing chain conversion

*

* This function aborts an ongoing chain of conversions.

* If scan(continuos) mode is selected, then bAllowRestart selects whether

* the abort command restarts the chain or stops it. For one shot conversion,

* this should be FALSE.

*

* @param[in] u32Instance - ADC instance number

* @param[in] bBlocking - wait and check that the ongoing conversion has stopped

* @param[in] bAllowRestart - restart continuos conversion instead of aborting

* @return status:

*  - ADC_SAR_IP_STATUS_SUCCESS: abort successful

*  - ADC_SAR_IP_STATUS_TIMEOUT: ongoing conversion could not be stopped

* @return void

*/

Adc_Sar_Ip_SetPresamplingSource

Prototype:

void Adc_Sar_Ip_SetPresamplingSource(const uint32 u32Instance,

const Adc_Sar_Ip_ChanGroupType pChanGroup,

const Adc_Sar_Ip_PresamplingSourceType pPresampleSource);

Description:

/*!

* @brief Set the Presampling Source for the channel group

*

* This function configures the Presampling Source for a channel group.

*

* @param[in] u32Instance - ADC instance number

* @param[in] pChanGroup - the channel group

* @param[in] pPresampleSource - the presampling source

* @return void

*/

Adc_Sar_Ip_EnableChannelPresampling

Prototype:

void Adc_Sar_Ip_EnableChannelPresampling(const uint32 u32Instance,

const uint32 u32ChnIdx);

Description:

/*!

* @brief Enable Presampling on one channel

*

* This function enables the Presampling on one channel of the ADC.

*

* @param[in] u32Instance - ADC instance number

* @param[in] u32ChnIdx - channel index(channel number in the RM)

* @return void

*/

Adc_Sar_Ip_DisableChannelPresampling

Prototype:

void Adc_Sar_Ip_DisableChannelPresampling(const uint32 u32Instance,

const uint32 u32ChnIdx);

Description:

/*!

* @brief Disable Presampling on one channel

*

* This function disables the Presampling on one channel of the ADC.

*

* @param[in] u32Instance - ADC instance number

* @param[in] u32ChnIdx - channel index(channel number in the RM)

* @return void

*/

Adc_Sar_Ip_EnablePresampleConversion

Prototype:

void Adc_Sar_Ip_EnablePresampleConversion(const uint32 u32Instance);

Description:

/*!

* @brief Enable Conversion Presampled Data

*

* This function enables bypass of the Sampling Phase, resulting in a conversion

* of the presampled data. This is available only for channels that have presampling

* enabled.

*

* @param[in] u32Instance - ADC instance number

* @return void

*/

Adc_Sar_Ip_DisablePresampleConversion

Prototype:

void Adc_Sar_Ip_DisablePresampleConversion(const uint32 u32Instance);

Description:

/*!

* @brief Disable Conversion of Presampled Data

*

* This function disables Sampling Phase bypass.

*

* @param[in] u32Instance - ADC instance number

* @return void

*/

Adc_Sar_Ip_EnableDma

Prototype:

void Adc_Sar_Ip_EnableDma(const uint32 u32Instance);

Description:

/*!

* @brief Enable DMA Requests

*

* This function enables requests to DMA from ADC

*

* @param[in] u32Instance - ADC instance number

* @return void

*/

Adc_Sar_Ip_DisableDma

Prototype:

void Adc_Sar_Ip_DisableDma(const uint32 u32Instance);

Description:

/*!

* @brief Disable DMA Requests

*

* This function disables requests to DMA from ADC

*

* @param[in] u32Instance - ADC instance number

* @return void

*/

Adc_Sar_Ip_EnableChannelDma

Prototype:

void Adc_Sar_Ip_EnableChannelDma(const uint32 u32Instance,

const uint32 u32ChnIdx);

Description:

/*!

* @brief Enable DMA on one channel

*

* This function enables DMA requests triggered by End of Conversion event from

* a selected channel.

*

* @param[in] u32Instance - ADC instance number

* @param[in] u32ChnIdx - channel index(channel number in the RM)

* @return void

*/

Adc_Sar_Ip_DisableChannelDma

Prototype:

void Adc_Sar_Ip_DisableChannelDma(const uint32 u32Instance,

const uint32 u32ChnIdx);

Description:

/*!

* @brief Disable DMA on one channel

*

* This function disables DMA requests triggered by End of Conversion event from

* a selected channel.

*

* @param[in] u32Instance - ADC instance number

* @param[in] u32ChnIdx - channel index(channel number in the RM)

* @return void

*/

Adc_Sar_Ip_DisableChannelDmaAll

Prototype:

void Adc_Sar_Ip_DisableChannelDmaAll(const uint32 u32Instance);

Description:

/*!

* @brief Disable DMA on all channels

*

* This function disables DMA requests triggered by End of Conversion event from

* all channels.

*

* @param[in] u32Instance - ADC instance number

* @return void

*/

Adc_Sar_Ip_SetDmaClearSource

Prototype:

void Adc_Sar_Ip_SetDmaClearSource(const uint32 u32Instance,

const Adc_Sar_Ip_ClearSourceType pDmaClear);

Description:

/*!

* @brief Set DMA Request Clear Source

*

* This function selects the DMA Request Flag Clear Source.

*

* @param[in] u32Instance - ADC instance number

* @param[in] pDmaClear - the clear source for DMA Requests (Ack from DMA Controller or

read of data registers)

* @return void

*/

Adc_Sar_Ip_SetWdgThreshold

Prototype:

void Adc_Sar_Ip_SetWdgThreshold(const uint32 u32Instance,

const uint8 u8RegisterIdx,

const Adc_Sar_Ip_WdgThresholdType * const pThresholdValues);

Description:

/*!

* @brief Configure watchdog threshold register

*

* This function configures the high/low thresholds for a certain register.

*

* @param [in] u32Instance - ADC instance number

* @param [in] u8RegisterIdx - the index of the register

* @param [in] pThresholdValues - the threshold values

* @return void

*/

Adc_Sar_Ip_SetConversionMode

Prototype:

void Adc_Sar_Ip_SetConversionMode(const uint32 u32Instance,

const Adc_Sar_Ip_ConvModeType eConvMode);

Description:

/*!

* @brief Configure conversion mode

*

* This function configures the used conversion mode.

* Note: The selected ADC instance must be in the IDLE state.

*

* @param [in] u32Instance - ADC instance number

* @param [in] eConvMode - selected conversion mode

* @return void

*/

Adc_Sar_Ip_SetCtuMode

Prototype:

Adc_Sar_Ip_StatusType Adc_Sar_Ip_SetCtuMode(const uint32 u32Instance,

const Adc_Sar_Ip_CtuModeType eCtuMode);

Description:

/*!

* @brief Configure CTU mode

*

* This function configures the CTU mode and enable.

*

* @param [in] u32Instance - ADC instance number

* @param [in] eCtuMode - selected CTU mode

* @return status:

*  - ADC_SAR_IP_STATUS_SUCCESS: set successful

*  - ADC_SAR_IP_STATUS_TIMEOUT: power up or down sequence timed out

*/

Adc_Sar_Ip_SetExternalTrigger

Prototype:

void Adc_Sar_Ip_SetExternalTrigger(const uint32 u32Instance,

const Adc_Sar_Ip_ExtTriggerEdgeType eTriggerEdge,

const Adc_Sar_Ip_ExtTriggerSourceType eTrggerSrc);

Description:

/*!

* @brief Configure external trigger

*

* This function configures the external trigger.

*

* @param [in] u32Instance - ADC instance number

* @param [in] eTriggerEdge - selected external trigger type

* @param [in] eTrggerSrc - selects normal, auxiliary normal or injected trigger

* @return void

*/

Adc_Sar_Ip_SetUserGainAndOffset

Prototype:

void Adc_Sar_Ip_SetUserGainAndOffset(const uint32 Instance,

const uint16 Gain,

const uint8 Offset);

Description:

/*!

* @brief Configure user gain and offset

*

* This function applies user gain and offset.

*

* @param [in] Instance - ADC instance number

* @param [in] Gain - Gain value

* @param [in] Offset - Offset value

* @return void

*/

Periodic Interrupt Timer

Pit_Ip_GetInterruptStatusFlag

Prototype:

boolean Pit_Ip_GetInterruptStatusFlag(uint8 instance, uint8 channel);

Description:

/*!
* @brief This function updates the value of UCRegB. It may be used to change duty cycle, phase shift or inserted dead time buffer with minimum overhead.

* @param        Instance          eMIOS hardware module index
* @param        Channel           The Channel in this eMIOS group
* @param        Value             The value to set
* @return       void
*/

Pit_Ip_GetLoadValue

Prototype:

uint32 Pit_Ip_GetLoadValue(uint8 instance, uint8 channel);

Description: N/A

Pit_Ip_Init

Prototype:

void Pit_Ip_Init(uint8 instance, const Pit_Ip_InstanceConfigType *config);

Description:

/**

* @brief         Function Name : Pit_Ip_Init

* @details       Driver initialization function. This function is called for each PIT hw Instance and

*                   - enables the functional clock for PIT Instance or for RTI_PIT

*                   - Sets PIT operation in Debug mode

*

* @param[in]     instance     PIT hw instance number

* @param[in]     configPtr    Pointer to a selected configuration structure

* @return        void

* @pre           The data structure including the configuration set required for initializing the driver

*/

Pit_Ip_InitChannel

Prototype:

void Pit_Ip_InitChannel(uint8 instance, const Pit_Ip_ChannelConfigType *chnlConfig);

Description:

/**

* @brief         Function Name : Pit_Ip_InitChannel

* @details       Initializes the PIT channels. This function should be called for each PIT hw channel and:

*                   - stops channel

*                   - disable interrupts

*                   - clears pending interrupts

*                   - initializes the state sructure for common process interrupt

*

* @param[in]     instance      PIT hw instance number

* @param[in]     chnlConfig    Pointer to a selected configuration structure.

* @return        void

* @pre           The data structure including the configuration set required for initializing the driver.

*/

Pit_Ip_Deinit

Prototype:

void Pit_Ip_Deinit(uint8 instance);

Description:

/**

* @brief         Pit_Ip_DeInit

* @details       De-Initializes the PIT instances. This functions is called for each PIT Instance and

*                    - resets all channels to default

*

* @param[in]     instance        Pit hw instance

* @return        void

* @pre           The data structure including the configuration set required for initializing the GPT driver.

*

*/

Pit_Ip_StartChannel

Prototype:

Pit_Ip_StatusType Pit_Ip_StartChannel(uint8 instance, uint8 channel, uint32 countValue);

Description:

/**

* @brief        Function Name : Pit_Ip_StartChannel

* @details      This function is called for starting the Pit timer channel.

*                    - sets the timeout value into the PIT timer channel register

*                    - enables the PIT channel

*

* @param[in]     instance        Pit hw instance

* @param[in]     channel         Pit hw channel

* @param[in]     countValue      channel timeout value

* @return        void

* @pre           The driver needs to be initialized. This function is called for starting the Pit timer channel.

*/

Pit_Ip_StopChannel

Prototype:

void Pit_Ip_StopChannel(uint8 instance, uint8 channel);

Description:

/**

* @brief         Function Name : Pit_Ip_StopChannel

* @details       This function is called for stopping the Pit counter. This function disables the PIT channel and:

*                    - stops PIT counter

*                    - clears interrupt flag

*

* @param[in]     instance        Pit hw instance

* @param[in]     channel         Pit hw channel

* @return        void

* @pre           The driver needs to be initialized. Pit_Ip_StartChannel must be call before.

*/

Pit_Ip_GetCurrentTimer

Prototype:

uint64 Pit_Ip_GetCurrentTimer(uint8 instance, uint8 channel);

Description:

/**

* @brief         Function Name : Pit_Ip_GetCurrentTimer

* @details       Gets counter value from CVAL Register.

*                The period range depends on the frequency of the PIT source clock

*

*

* @param[in]     instance        Pit hw instance

* @param[in]     channel         Pit hw channel

*

* @return        counterValue    counter value

* @pre           The driver needs to be initialized.

*/

Pit_Ip_EnableChannelInterrupt

Prototype:

void Pit_Ip_EnableChannelInterrupt(uint8 instance, uint8 channel);

Description:

/**

* @brief        Function Name : Pit_Ip_EnableChannelInterrupt

* @details      This function enables the IRQ corresponding to the Pit timer channel

*

*

* @param[in]    instance        Pit hw instance

* @param[in]    channel         Pit hw channel

* @return       void

* @pre          The driver needs to be initialized.

*/

Pit_Ip_DisableChannelInterrupt

Prototype:

void Pit_Ip_DisableChannelInterrupt(uint8 instance, uint8 channel);

Description:

/**

* @brief        Function Name : Pit_Ip_DisableChannelInterrupt

* @details      This function disables the IRQ corresponding to the Pit timer channel

*

*

* @param[in]     instance        Pit hw instance

* @param[in]     channel         Pit hw channel

* @return        void

* @pre           The driver needs to be initialized.

*/

Pit_Ip_SetLifetimeTimer

Prototype:

void Pit_Ip_SetLifetimeTimer(uint8 instance);

Description:

/**

* @brief        Function Name : Pit_Ip_SetLifetimeTimer

* @details      Build the 64-bit lifetimer.

*               The lifetime timer is a 64-bit timer which chains timer channel 0 and timer channel 1 together

*               with the start value of both channels is set to the maximum value(0xFFFFFFFF).

*               The period of lifetime timer is equal to the "period of

*               timer 0 * period of timer 1".

*

* @param[in]    instance        Pit hw instance

* @return       void

* @pre          The driver needs to be initialized.

*/

Pit_Ip_GetLifetimeTimer

Prototype:

uint64 Pit_Ip_GetLifetimeTimer(uint8 instance);

Description:

/**

* @brief         Function Name : Pit_Ip_GetLifetimeTimer

* @details       Read current lifefime counter value.

*                The Lifetime timer is 64-bit timer which chains timer 0 and timer 1 together.

*                The period of lifetime timer equals to "period of timer 0 * period of timer 1".

*                This feature returns an absolute time stamp in count. The time stamp

*                value does not exceed the timer period. The timer is down-counting.

*

*

*

* @param[in]     instance        Pit hw instance

*

* @return        lifeTimeValue

* @pre           The driver needs to be initialized.

*/

Configuration

This section will illustrate the GUI settings for the NXP Configuration Tool which brings in the components available using NXP’s RTD SDK for the S32K344 SoC.

AUTOSAR Fee

Cache Ip

Dma Ip

IntCtrl Ip

FlexCAN Ip

FlexCAN Device name

FlexCAN Device 0 configuration

FlexCAN Device 1 configuration

FlexCAN Device 2 configuration

FlexCAN General

FlexCAN Timeout Configuration

Lpspi Ip

Rm

All other RmConfigSet tabs are “default” settings – not important.  The Rm component is primarily used to configure the DMA MUX to connect the eDMA to various sources.

Siul2 Dio

Siul2 Port

Siul2_Icu

Pit

ADC SAR

Calibrations

N/A

Sequence Diagrams

N/A

Source File List and Description

The following sections outline the source files needed to realize the S32K344-MCAL Component within a target application that becomes linked in a build procedure.

The ABS Development team uses NXP’s downloadable RTD (Real-Time Drivers) SDK.  This SDK supports AUTOSAR and non-AUTOSAR applications which target ARM Cortex-M cores and ISO 26262 compliance for all software layers.  Please refer to this link to understand more about the SDK:

https://www.nxp.com/design/design-center/software/automotive-software-and-tools/real-time-drivers-rtd:AUTOMOTIVE-RTD

This RTD SDK is integrated into the S32DS IDE where the NXP Configuration Tool may be accessed (*.mex file is a project for this tool).  Within the IDE, users may pick and choose the components to add for generation and integration for an application.

The ABS Software Team uses RTD SDK version as follows:

S32K3_S32M27x Real-Time Drivers ASR R21-11 Version 6.0.0

This can be downloaded on NXP’s site if a developer has a user account with NXP established.

Source Files

Below is an image that illustrates the directories involved with the component.  Since the MCAL Component is primarily generated files, further detail will be implored into this where necessary.

Directory: board

The ‘board’ directory contains generated files that configure the GPIO (SIUL2) pins and board-specific information.

Directory: generate

This directory contains source files that configure how the (static) driver sources are to be employed.  For instance, a configuration that sets up specific CAN or SPI peripheral settings exist here.  Most of the time, these configurations are in a const structure of some sort where the data is directly aligned with the settings in the NXP Config Tool GUI.  There will be an ‘include’ directory for header files and a ‘src’ directory for the const structs.

Directory: mcal-base

The ‘mcal-base’ directory contains files that are used by all RTD files.  These are base RTD SDK files that are placed here to have all developers reference these files in a common location of this repo.

Directory: Project_Settings

The ‘Project_Settings’ folder has several MCU specific settings.  This directory contains the linker script for a target application to use when it constructs a file to flash.  Also, startup code is contained here which includes basic system initializations along with an interrupt vector table that can be modified to add ISRs as needed by a target application.  These files are only generated once the project is initially constructed; a typical generation process will not recreate these.  Hence, these may be modified in other versions of the MCAL component.

Directory: RTD

The ‘RTD’ directory contains the static files that are “the drivers” of the S32K344 SoC.  These files are 100% generated and should not be modified for any reason.  As developers include more peripherals in the NXP Config Tool, these files will be added as necessary.  Likewise, if peripheral components are removed, the static files here will also be removed from the generation process.

CMakeLists.txt

This file will be used by the target application to integrate the S32K344-MCAL Component into it.  There should not be much reason to modify this file since the sources are again, primarily generated and the CMakeLists.txt file will “glob” the sources (i.e., *.c, *.s and *.h files) it needs to build the component.

Build and Test

The process to generate the S32K344-MCAL Component sources can be achieved in a few steps.  It is important to follow these as to not have difficulties working with the NXP Config Tool which is part of the S32DS IDE.  The following steps must be followed to generate successfully:

- Check out the ‘develop’ branch (HEAD of ‘develop’) or feature/bugfix that has ‘develop’ as a parent (intentions of this ‘develop’ branch are to not only generate files for releases, but to also “smoke” test if integration will be successful).

- In the root directory of the MCAL Component, run the batch script called ‘prepare-for-generation.bat’.  This will delete all files that may exist that are generated and not tracked.  As well, it clears out the contents of the ‘src’ folder – which are files that are released (in ‘main’ that are used by a target application).

- In the S32DS Project Explorer window, ensure that the contents are refreshed – very important!  



Notice the differences after a refresh:

- Make the changes to the drivers in the NXP Config Tool.  



Select the driver to configure and the configurable items will appear on the right-hand side – such as in the image the “BaseNXP Configuration” driver shown above.

- Press the “Update Code” button, and all the needed RTD files (drivers) will be added to the S32DS project along with the generated configuration (driver config).

- Test build the S32DS project.  If successful, go to next step.  If the build fails, address the issues and find a way to make build succeed.

- Back in the root of the MCAL Component, run the batch script called “copy-dirs.bat”.  This will copy the generated contents into the ‘src’ folder.

- Optionally, a developer may want to commit the changes if necessary to preserve the work done.

- The final step to verify the configuration is preliminarily acceptable is to build the component via CMake and link it to some generic “main.c” only to be able to do a complete build (i.e., compilation and linking) with the MCAL Component.  Again, this must succeed at this point in time, or the chances of success that at the time this MCAL Component is to be integrated into a target application will be low.

- If building the component via CMake is successful, the process to build and test has been achieved successfully.  The chances of integration of the MCAL Component succeeding in a target application environment are relatively high.

Integration Procedure

Latest changes for the S32K344-MCAL Component are available in ‘main’ branch which are tagged with labels per the ABS Configuration Management document.  Integration into a target application can be achieved byb following the following steps:

- Add the component as submodule to the application repository with command git submodule add -f https://<user>@bitbucket.org/americanbatterysolutions/<repo_name> from the directory where this component reside.

- Init and update the submodule by running git submodule update --recursive –init

- Specific version of the component can be chosen by running the cmd git checkout <tag/branch/commit> from the submodule directory.

- Make sure all the dependent submodules are present in the application (e.g. S32K344-FreeRTOS for the S32K344-MCAL).

- Ensure that in a target application’s “main” CMakeLists.txt file (this should be in root directory of CMake-managed project) adds the subdirectory of the S32K344-MCAL Component.  Also, submodules of dependencies will have their sub-directories added likewise, but the image below illustrates how to add in specifically the S32K344-MCAL Component alongside other CMake objects.

- If all is done correctly, successful building of the target application via CMake should be achieved and the MCAL APIs are linked into the build.

Resource Consumption

| Resource | Usage |
| --- | --- |
| RAM | TBD |
| ROM | TBD |
| CPU Bandwidth | TBD |

MISRA + BARR-C

N/A

References

N/A